A. IMMEDIATE CAUSES – ACTIONS (Unsafe Acts)

A1. PROCEDURE & RULE VIOLATIONS

A1.1 Individual Rule Violation

The person knowingly and deliberately chose to violate an established safety rule. The individual was aware of the rule, had received training, had access to the procedure, and made a conscious decision to deviate from it. This is an intentional act, not a knowledge gap or a capability failure.

Choose if: The worker knew the rule, had been trained on it, the rule was accessible, and the violation was deliberate.
→ Typical: Skipping LOTO steps before working on a pressurized line on an oil/gas site, not wearing an H₂S detector despite it being mandatory and available on a gas field, bypassing the pre-blast evacuation procedure in a mine, not using a harness and helmet during elevated work on a construction site despite both being available.

Not this if:

- The rule was unknown to the worker → D9.3 (SPP communication/access ineffective)

- No training was ever provided → D3.1 (Training not provided)

- The rule is impractical or impossible to apply in the field → D9.4 (SPP applicability weak)

- More than one person committed the same violation → A1.2 (Group/team rule violation)

- The violation has become normalized across the team → D1.5 (Normalization of deviance)

- Management is aware and tolerates or encourages the violation → D1.9 (Management tolerance of known deviations)

- The supervisor is unaware the violation is occurring → D1.2 (Inadequate supervision)

- No monitoring or audit of procedure compliance exists → D9.5 (Monitoring/audit inadequate)

A1.2 Group / Team Rule Violation

Multiple people collectively decided to violate a known safety rule. Group dynamics or cultural norms are the determining factor, not individual choice alone.

Choose if: More than one person participated in the decision to knowingly violate an established safety practice.
→ Typical: An entire shift crew performing hot work without obtaining a permit at an oil refinery, a mine team entering a confined space without gas measurement because "everyone does it," a construction crew skipping a safety briefing under collective production pressure.

Not this if:

- Only one person is involved → A1.1 (Individual rule violation)

- A supervisor directed or ordered the violation → A1.3 (Supervisor rule violation)

- The behavior reflects peer pressure or "normal for everyone" thinking → D1.5 (Normalization of deviance)

- Management is aware and has not intervened → D1.9 and D1.3

- Unsafe behavior was rewarded or left uncorrected → C3.5 (Wrong behavior rewarded)

- Procedure compliance is not monitored → D9.5

- The issue points to an organizational/cultural problem → D1 and D2 sections

- Safety leadership is absent → D1.8 (Insufficient visible leadership)

A1.3 Supervisor / Manager Rule Violation

A manager or supervisor personally violated a safety rule OR directly instructed others to violate it. The defining feature is that the breach originates from leadership, either through direct action or through a directive.

Choose if: A person in a leadership role directly violated a rule or issued an instruction to others to do so.
→ Typical: A supervisor saying "skip the permit, we're in a hurry" on an energy site, a drilling supervisor commissioning a line without a required pressure test on an oil field, a mine shift foreman ordering a blast without completing evacuation, a construction site foreman instructing workers to skip scaffold safety checks to save time.

Not this if:

- The supervisor was unaware the violation was occurring → D1.2 (Inadequate supervision)

- The violation was individual and not driven by a leadership directive → A1.1

- Organizational pressure is the underlying driver → D1.4 (Production pressure)

- Management tolerance has become systemic → D1.9

- Accountability mechanisms are absent → D1.3

- If a manager consciously onboarded an unqualified contractor under schedule or cost pressure → D7.3

A1.4 Procedure Not Understood

The procedure exists but the worker was unable to comprehend it due to a language barrier, excessive technical complexity, or conflicting instructions. This is a perception and comprehension issue, not deliberate non-compliance.

Choose if: The procedure exists but the worker genuinely could not understand it.
→ Typical: A foreign-national contractor unable to understand an isolation procedure written only in English on an oil/gas site, a miner misinterpreting a blasting procedure filled with technical jargon, a new worker on a construction site unable to correctly complete a complex work permit form.

Not this if:

- No procedure exists at all → D9.1 (No SPP for task)

- The training system failed to build comprehension → D3.2 or D3.3

- The procedure is physically impossible to follow in the field → A1.5

- The language barrier is a systemic organizational issue → D2.7 (Information misunderstood)

A1.5 Procedure Exists But Not Applicable in Field

The procedure exists but is physically impossible to execute as written; it contains unrealistic assumptions about time, equipment availability, or spatial conditions. The worker attempted to follow it but could not, due to design flaws in the procedure itself.

Choose if: The worker tried to follow the procedure but was unable to, due to the procedure's own design flaws.
→ Typical: The tool specified in a procedure physically not fitting between pipes on an energy site, an isolation valve referenced in an oil platform procedure not actually existing in the field, the safe stand-off distance required by a mine blasting procedure being physically unavailable in the tunnel geometry, the lifting equipment specified in a construction procedure being unable to operate in that section of the site.

Not this if:

- No procedure exists at all → D9.1

- The procedure could not be understood → A1.4

- A modification was made without updating the procedure → D4.3 (MoC ineffective)

- The issue is an ergonomic or spatial design problem → D5.3 (Ergonomic design errors)

A2. TOOLS, EQUIPMENT & VEHICLE USE

A2.1 Wrong Use of Equipment / Tool / Vehicle

Equipment has been used entirely outside its designed purpose. The user either could not access the correct equipment or opted for the nearest available alternative to complete the task. This category covers fundamental functional mismatches between the equipment and the task.

Choose if: The type of equipment used is fundamentally wrong for the task by design, and the person made this choice knowingly.
→ Typical: Using a forklift arm as a working platform on an oil/gas site, using a steel drum as a ladder in a mine, using a crane rated only for lifting to drag horizontal loads at a power plant, using an under-capacity compressor for pressure testing on a construction site.

Not this if:

- The correct equipment was unavailable or could not be sourced → B2.1 (Suitable equipment not available)

- The person did not know which equipment was correct → C3.2 (Insufficient skill practice) or D3.1 (Training not provided)

- The equipment type is correct but the method is wrong → A2.2 (Improper tool use)

- Equipment selection is ambiguous in the procedure → D9.2 (SPP development ineffective)

- No procedure defines the correct equipment → D9.1 (No SPP for task)

- Risk assessment did not address equipment selection → D4.1

- Ergonomic constraints prevent correct equipment from fitting the workspace → D5.3 or B4.1

A2.2 Improper Tool Use

The tool type is appropriate for the task, but the method, technique, or application is unsafe. The problem lies not in the tool itself but in how it was used.

Choose if: The tool type was suitable for the task, yet the person applied the wrong technique or method.
→ Typical: Over-torquing a pipe fitting with a power tool on an oil site where hand-tightening was required, creating an opening with a cutting disc instead of a drill in a mine, working on a live energy line with a standard tool instead of an insulated one at a power plant, using a screwdriver as a chisel on a construction site.

Not this if:

- The tool type is fundamentally wrong → A2.1 (Wrong use of equipment)

- The person did not know the correct technique → C3.2 (Insufficient skill practice) or D3.1 (Training not provided)

- How the tool should be used is not defined in the procedure → D9.1 or D9.2

- Instructions were unclear or impractical → A1.4 or D9.4

- No tool usage instruction was ever provided → D3.2 (Training design ineffective)

- Field coaching on correct technique was never done → C3.3 (Skill coaching not done)

A2.3 Using Equipment with Known Defect

The equipment was visibly defective or its defect was known to the user, yet operation continued. The core issue is that a known risk was deliberately accepted.

Choose if: The defect or damage was VISIBLY present and the user was aware of it.
→ Typical: Continuing to operate a pressure pump tagged "out of service" on an oil/gas site, lifting a load with a cracked hoist chain in a mine, driving a vehicle with known brake degradation at a power plant, using a damaged scaffold connector without replacement on a construction site.

Not this if:

- The defect was hidden or an internal failure → B2.1 (Equipment failure – unexpected) or B2.7 (Corrosion/erosion/fatigue)

- The defect was reported but not rectified → D6.6 (Deferred maintenance normalization)

- The inspection system failed to detect the defect → D6.3 (Inspection/calibration inadequate)

- Defect was performance-based and not visually apparent → B2.1

- Management was informed but took no action → D1.9 and D6.6

- Use of defective equipment was normalized across the group → A1.2 and D1.5

A2.4 Using Tool with Known Defect

The worker observed the tool to be defective or damaged yet continued to use it. Unlike A2.3, this concerns hand tools or portable equipment rather than fixed plant equipment.

Choose if: The worker personally saw the defect and continued using the tool.
→ Typical: Using an electrical cable with exposed conductors on an oil site, using a battery charger with a broken protective cover in a mine, using a measuring instrument with partially cracked insulation at a power plant, continuing to use a damaged impact drill on a construction site.

Not this if:

- The defect was not visible or was an internal failure → B2.4 (Inadequate tool preparation) or B2.3 (Tool failure)

- No tool inspection program exists → D6.3 (Inspection/calibration inadequate)

- Multiple people are ignoring the same issue → A1.2 and D1.5

- No reporting mechanism exists → D1.7 (Weak reporting/learning culture)

- Accountability for defect reporting is absent → D1.3

A2.5 Improper Positioning / Storage of Tools / Equipment

Equipment, vehicles, or materials have been placed or stored in a way that creates a risk of tipping, falling, striking, or blocking access.

Choose if: The storage or positioning decision created a hazard and that decision belonged to the individual.
→ Typical: Leaving hand tools at the edge of scaffold on an offshore platform, stacking chemical drums on uneven ground causing tip-over in a mine, creating a material pile that blocks an emergency exit at a power plant, parking a machine on a slope without applying the handbrake on a construction site.

Not this if:

- Disorganization stems from physical workspace design → B4.1 (Cramped layout/poor arrangement)

- No storage procedure exists → D9.1 (No SPP for task)

- The storage rules were never taught → D3.1 (Training not provided)

- Supervision absence allows this pattern to persist → D1.2 (Inadequate supervision)

- Poor layout is a design issue → D5.3 (Ergonomic design errors)

- Routine housekeeping is not maintained → B4.7 (Poor housekeeping)

A2.6 Operating Outside Design Limits

The operator deliberately exceeded the system's or equipment's known limit values (pressure, speed, load, temperature, etc.).

Choose if: It can be clearly established that the operator knew the limit and intentionally exceeded it.
→ Typical: Raising pipeline pressure above the maximum allowable operating pressure on an oil site, overloading a conveyor belt beyond its nominal capacity in a mine, running a turbine above its maximum rated speed at a power plant, exceeding the rated lift capacity of a tower crane on a construction site.

Not this if:

- Limits were never taught to the worker → D3.1 (Training not provided)

- Limits are ambiguous or contradictory in the procedure → D9.2 (SPP development ineffective)

- The limit exceedance is driven by production pressure → D1.4 (Production pressure)

- Design limits do not reflect real operational conditions → D5.2 (Design inputs wrong)

- Limit exceedance has become systematically accepted → D1.9 (Management tolerance of known deviations)

- Management is unaware and no monitoring exists → D1.2 and D9.5

A2.7 Temporary Modification / Jury-Rigging

An unauthorized, non-standard temporary repair or modification has been made to bring defective or incomplete equipment back into use. Such improvised solutions may appear functional in the short term but carry serious safety risks.

Choose if: The worker applied an unauthorized, temporary, and unsafe repair or modification.
→ Typical: Temporarily sealing a flange leak with tape and clamps to continue production on an oil/gas site, securing a broken door latch with wire in a mine, creating a wire bridge to compensate for insufficient voltage protection at a power plant, tying off a damaged platform with steel rope to continue lifting on a construction site.

Not this if:

- The modification was authorized and approved → D4.3 (Change management/MoC ineffective)

- The temporary fix has long been treated as permanent → D4.6 (Temporary risk controls treated as permanent)

- Absence of a maintenance system drove the person to this approach → D6.1 (Maintenance strategy inadequate)

- Deferred maintenance made it unavoidable → D6.6 (Deferred maintenance normalization)

- The practice is normalized across the group → D1.5 or D1.9

A2.8 Process Control Errors – Operator Misoperation

The operator incorrectly managed the sequential steps, parameter inputs, or dynamic control of a process system. This error may not be intentional, yet it constitutes a direct hazard-producing action.

Choose if: The operator followed the wrong valve sequence, entered incorrect parameters, or mismanaged process flow dynamics.
→ Typical: Opening valves in the wrong sequence during crude oil line commissioning at a refinery, entering the wrong air/fuel ratio for a combustion chamber at a gas plant, doubling the chemical dosage setting at a mineral processing facility, energizing circuit breakers without synchronization at a power plant.

Not this if:

- The procedure is ambiguous or contradictory → A1.4 (Procedure not understood) or D9.2

- Control interface design systematically promotes error → B4.8 or D5.3

- Inadequate training is the root cause → D3.2 (Training design ineffective)

- The automation system provided incorrect information → B4.9 (Workplace indicators inadequate)

- Fatigue or distraction is the primary contributing factor → C1.4 or A4.1

- The error was unintentional and reflex-based → A4.5 (Slip/lapse)

A3. PROTECTIVE EQUIPMENT (PPE) & METHODS

A3.1 PPE / Method Need Not Recognized

The worker was unaware that PPE was required for the task or environment. This is not deliberate non-compliance; it reflects a gap in hazard awareness or risk assessment knowledge.

Choose if: The worker genuinely did not know the hazard existed or that PPE was required.
→ Typical: Not knowing that respiratory protection is required in a newly opened H₂S-risk zone on an oil/gas site, being unaware of the dermal absorption risk of a newly introduced chemical in a mine, not measuring noise levels and assuming hearing protection is unnecessary at a power plant, not knowing that cutting silica-containing material requires a respirator on a construction site.

Not this if:

- The person knew but chose not to use PPE → A3.2 (Not using required PPE/methods)

- The risk assessment did not identify this hazard → D4.1 (Risk assessment inadequate)

- The PPE requirement is not written in any procedure → D9.1 (No SPP for task)

- Training never covered this hazard → D3.1 (Training not provided) or D3.2

- A new hazard was introduced without assessment → D4.3 (MoC) or D4.1

- Hazard information was not communicated to the worker → D2.2 (Vertical communication inadequate)

A3.2 Not Using Required PPE / Methods

PPE was available, training had been provided, and the requirement was known — yet the worker deliberately chose not to use it. This is a conscious violation, not a knowledge or access problem.

Choose if: PPE was provided, the worker was trained, and the requirement was known.
→ Typical: Working without wearing a gas detector that is available and clipped to the vest on an oil site, entering a blast exclusion zone without a helmet in a mine, removing insulated gloves mid-task during high-voltage testing at a power plant, performing cutting work with safety glasses stored in a pocket on a construction site.

Not this if:

- PPE was not available → A3.4 (PPE/methods not available)

- The requirement was unknown → A3.1

- PPE was used incorrectly → A3.3

- This behavior is normalized across the group → A1.2 or D1.5

- Management is aware of the non-compliance and tolerates it → D1.9

- PPE is uncomfortable or impractical for the task → D9.4 (Applicability weak)

- No monitoring or enforcement mechanism exists → D9.5 or D1.2

A3.3 Wrong Use of PPE / Methods

PPE was used, but either the wrong type was selected or the correct type was worn incorrectly. As a result, the PPE failed to provide the intended protection.

Choose if: PPE was used but in the wrong category for the hazard, or worn in a manner that rendered it ineffective.
→ Typical: Wearing a filter half-mask instead of SCBA in an IDLH H₂S atmosphere on an oil/gas site, using leather gloves with no chemical resistance for acid contact in a mine, using a low-category dielectric helmet during high-voltage testing at a power plant, anchoring a fall arrest harness to a pipe with no load-bearing capacity on a construction site.

Not this if:

- PPE was not used at all → A3.2

- PPE specification was wrong from the outset → A3.7 (PPE selection mismatch with hazard severity)

- The correct PPE usage was never taught → C3.2 (Insufficient skill practice) or D3.2

- Wrong PPE is systematically distributed → B1.6 (Wrong PPE specified)

- Risk assessment did not identify the correct PPE → D4.1

- PPE selection criteria are missing from the procedure → D9.2

A3.4 PPE / Methods Not Available

The worker knew PPE was required but was unable to obtain it; it was out of stock, delivery was delayed, or the correct size was unavailable.

Choose if: The worker knew PPE was needed but physically could not obtain it.
→ Typical: Respiratory equipment stock depleted for an H₂S zone on an oil site, blast-resistant visors delayed in the procurement pipeline at a mine, correct voltage-class gloves unavailable in the required size at a power plant, fall arrest equipment absent due to budget exclusion on a construction site.

Not this if:

- PPE was available but not used → A3.2

- Resource allocation is inadequate at leadership level → D1.1 (Weak leadership commitment)

- The PPE requirement was never identified → D4.1 or D9.1

- Budget constraint is a systemic organizational issue → D1.1

A3.5 Safety Devices / Warning Systems Removed or Bypassed

The worker or team deliberately disabled an existing safety device or warning system in order to proceed with the work. This requires an active, conscious decision specific to this task.

Choose if: An active, deliberate bypass decision was made specifically for this work.
→ Typical: Skipping the LOTO lock to begin work under energy on an oil/gas site, disabling a gas alarm sensor to avoid a production shutdown in a mine, jumpering a protective relay during testing at a power plant, bridging a control cable to skip a safety interlock on a construction site.

Not this if:

- The device was previously removed and never reinstalled → A3.6 (Guards/warning systems previously removed)

- Bypass is normalized across the group → D1.5 or D1.9

- Management is aware of the practice → D1.9

- System design makes bypass attractive or necessary → D5.3 (Ergonomic design errors)

- No monitoring or enforcement system exists → D9.5

- LOTO procedure itself is inadequate → D4.5 (Energy isolation/LOTO ineffective)

A3.6 Guards / Warning Systems Previously Removed

A safety device or guard was removed at some point in the past and was never reinstalled. This is not a decision made for the current work; the current situation is the enduring consequence of a past failure.

Choose if: Removal occurred during a previous activity and the device has remained absent since.
→ Typical: A pressure relief valve guard removed last month still missing on an oil site, a barrier system not reinstalled after maintenance in a mine, warning lights broken and unrepaired for months at a power plant, platform guardrails temporarily removed and never remounted on a construction site.

Not this if:

- Deliberate bypass was made for this specific job → A3.5

- Maintenance system failure is the root cause → D6.2 (Maintenance execution inadequate)

- Absence was not noticed due to lack of field supervision → D1.2 (Inadequate supervision)

- No site tours are conducted and deficiencies go undetected → D1.8 (Insufficient visible leadership)

- Inspection program is not functional → D6.3

- Accountability mechanism does not exist → D1.3

A3.7 PPE Selection Mismatch with Hazard Severity

The PPE type or protection level specified during risk assessment or in the procedure does not match the actual magnitude of the hazard. The problem lies in the PPE specification process, not in the individual's behavior.

Choose if: The PPE specification itself is inadequate to protect against the actual risk level.
→ Typical: Specifying only a filter half-mask against IDLH H₂S concentrations on an oil/gas site, applying a light-class helmet standard in a heavy rock fall zone in a mine, selecting low-category dielectric gloves for high-voltage test environments at a power plant, specifying an under-rated respiratory protection device for explosive-dust environments on a construction site.

Not this if:

- Risk assessment never identified this hazard → D4.1 (Risk assessment inadequate)

- Training did not teach correct PPE selection → D3.2

- The procurement system delivered the wrong product → D8.1 or D8.2

- Individual selection error occurred → A3.3 (Wrong use of PPE/methods)

- The PPE standard is outdated or not revised → D9.2 (SPP development ineffective)

- The person specifying PPE lacked sufficient expertise → C3.1 (Required skill assessment inadequate)

A4. HUMAN ERROR, ATTENTION & BEHAVIORAL GAPS

A4.1 Distraction / Attention Diverted

The worker's attention was pulled away from the task being performed, resulting in a critical step being missed, a wrong action taken, or a hazard going unnoticed. The distraction source is external: noise, phone calls, visitors, alarm habituation, or multitasking pressure.

Choose if: An external factor diverted attention at a critical moment and this is the direct trigger of the incident.
→ Typical: A control room operator receiving a phone call mid-way through a critical valve sequence on an oil/gas site, a low-priority alarm that has rung so often it is ignored causing a real hazard signal to be missed in a mine, a visitor group bombarding a panel technician with questions at a power plant, a danger warning command not being heard amid heavy vehicle traffic noise on a construction site.

Not this if:

- Workspace design systematically creates distraction → B4.8 (Workplace design increases error likelihood)

- Gauges, alarms, or labels provide inadequate information → B4.9 (Workplace indicators inadequate)

- Fatigue or sleep deprivation has reduced attention capacity → C1.4 (Fatigue)

- Working hours and workload are driven by organizational pressure → D1.4 (Production pressure)

- Alarm habituation is a design-level problem → D5.3

- Attention limitations are a permanent individual characteristic → C2.1 (Memory or attention limitations)

A4.2 Environmental Hazards Not Noticed

The worker passed by or approached a physically visible hazard without noticing it. The hazard itself was not hidden; the issue is a gap in situational awareness.

Choose if: The hazard was visible and the person passed by or approached it without perceiving it.
→ Typical: Stepping into an oil-water puddle on a process site, walking past a cracked and loosened rock block in a mine, failing to read a warning sign next to an open hatch at a power plant, tripping over a metal profile left in the work zone on a construction site.

Not this if:

- The hazard was physically concealed or invisible → B3.x (Unexpected exposure to hazardous energy/material)

- Fatigue or sleep deprivation reduced awareness → C1.4 (Fatigue)

- Attention capacity is permanently limited → C2.1 (Memory or attention limitations)

- Insufficient lighting prevented the hazard from being seen → B4.2 (Inadequate lighting)

- Missing or illegible signage contributed to non-detection → B4.6 (Poor/illegible labeling/signage)

- Multiple monitoring demands caused it to be missed → A4.7 (Task complexity exceeding human capacity)

A4.3 Inappropriate / Unsafe Workplace Behavior

The individual or individuals engaged in deliberate and dangerous behavior driven by entertainment, excitement, or group dynamics. This is a conscious choice, not a performance pressure outcome.

Choose if: The behavior was intentional, for amusement, as a dare, or to provoke a reaction — and was clearly unsafe.
→ Typical: Running or pushing each other in a hazardous zone on an offshore platform, "testing" how close one can stand before a blast in a mine, approaching a high-voltage area as a dare at a power plant, shaking scaffolding to frighten a colleague on a construction site.

Not this if:

- Absence of supervision enables this behavior → D1.2 (Inadequate supervision)

- Accountability mechanisms are non-functional → D1.3 (Lack of accountability)

- Unsafe behavior has become a group norm → D1.5 (Normalization of deviance)

- Leadership observes such behavior and fails to intervene → D1.9 or D1.10

- Psychological safety issues drive erratic behavior → D1.11

- Cultural breakdown is systemic → D1 and D2 sections

A4.4 Failure to Warn Others

The worker noticed a hazard that could harm others but did not issue a warning, place barriers, or report it. This passive failure creates a chain vulnerability.

Choose if: The person personally saw or detected the hazard but did not communicate it.
→ Typical: Quietly passing by a small pipe leak to avoid a production shutdown on an oil site, a miner noticing rock cracking but failing to pass it on at shift handover, a technician seeing a defective component at a power plant and walking past without raising a work order, passing by a loose anchor point with fall risk on a construction site without warning anyone.

Not this if:

- Psychological safety prevents speaking up → D1.11 (Absence of psychological safety)

- Reporting channels are unknown or non-existent → D1.7 (Weak reporting/learning culture)

- Production pressure discouraged the warning → D1.4 (Production pressure)

- Vertical or horizontal communication mechanisms are dysfunctional → D2.1 or D2.2

- Past experience suggests reporting yields no result → D1.3 (Lack of accountability)

- The hazard was genuinely not noticed → A4.2 (Environmental hazards not noticed)

A4.5 Unintentional Human Error (Slip / Lapse)

The person did not deliberately violate a rule; an involuntary error occurred at the level of perception, memory, or motor action. This is not negligence — it reflects genuine human cognitive limitations.

Choose if: The error was unintentional, involuntary, and occurred while the person was genuinely trying to do the right thing.
→ Typical: Activating the wrong button of two adjacent controls in a control room on an oil/gas site, a mine operator skipping a step and moving on to the next item on a checklist, accidentally reading a temperature gauge instead of a pressure gauge at a power plant, inadvertently closing the open valve of two visually similar valves on a construction site.

Not this if:

- The shortcut was deliberate → A1.1 (Individual rule violation)

- Design errors systematically make this type of error likely → B4.9 or D5.3

- Fatigue, chronic sleep deprivation, or shift rhythm disruption is a contributing factor → C1.4 or C1.6

- Memory or attention capacity creates a permanent limitation → C2.1

- Procedure complexity makes error practically inevitable → D9.2 or D9.4

- The person was under the influence of substances → C1.5

A4.6 Automatic / Routine Actions Without Conscious Control

The worker has entered an "autopilot" mode produced by long-established routine; as a result, in a different environment, a different task, or a modified setting, old habits were automatically triggered. This is a skill-based error category.

Choose if: The person believed they were performing a familiar routine but was actually in a different task or modified environment, and applied the old routine automatically.
→ Typical: A petro-chemical operator unconsciously applying the valve sequence of a previous unit after being transferred to a new one, a mine machine operator making an automatic wrong turn at a familiar junction despite being redirected to a different route, closing the wrong of two circuit breakers post-maintenance based on remembered prior positions at a power plant, applying the old mix ratio by habit instead of the revised one during concreting on a construction site.

Not this if:

- Workspace or control design systematically produces this confusion → B4.9 or D5.3

- The change was never communicated to the worker → D4.3 (MoC) or D2.2 (Vertical communication inadequate)

- Inadequate training is the root cause → D3.2

- Fatigue or cognitive depletion is a facilitating factor → C1.4

A4.7 Task Complexity Exceeding Human Capacity

The task or process places a demand for simultaneous information processing, attention, or decision-making that exceeds what any average capable individual can safely manage. This category reflects a human factors gap in task design, not a deficiency in the individual's competence.

Choose if: The concurrent demands of the task exceed what any reasonably competent person could safely manage.
→ Typical: An operator monitoring over 20 alarm screens simultaneously during a maintenance window on an oil refinery, a mine process controller being assigned sole responsibility for 15 interacting parameters, an emergency procedure requiring 12 simultaneous steps at a power plant, a single supervisor required to coordinate 5 sub-contractors at the same time on a construction site.

Not this if:

- Design creates a systemic cognitive overload → D5.3 (Ergonomic design errors)

- Insufficient staffing is the cause → D3.5 (Critical competency gaps)

- Task load analysis was never performed → D4.1 (Risk assessment inadequate)

- The individual lacked training to manage complexity → D3.2

- Production pressure drove the overload → D1.4

A4.8 Time Pressure Leading to Cognitive Shortcuts

The worker, under deadline pressure, shift-end urgency, or management pressure, deliberately skipped or abbreviated critical safety steps in order to complete work faster. This is a conscious shortcut decision made under pressure.

Choose if: It can be clearly established that the person skipped specific steps due to time pressure.
→ Typical: Skipping the LOTO verification step because "there's no time" before a maintenance window closes on an oil/gas site, entering a confined space without gas measurement to catch the end of a shift in a mine, leaving a planned safety test incomplete as the commissioning deadline runs out at a power plant, shortening concrete curing time to avoid missing a delivery date on a construction site.

Not this if:

- Pressure originates systematically from management → D1.4 (Production pressure)

- Unrealistic scheduling is a systemic planning failure → planning/D4.1

- The individual chose to violate through preference rather than pressure → A1.1 (Individual rule violation)

- Time pressure became a group decision → A1.2

- Procedure design forces shortcuts by being impractical → D9.4 (Applicability weak)

- Absence of psychological safety prevents the worker from saying "no" → D1.11

A4.9 Team Coordination Failure

Team members executing the same task or interconnected tasks failed in planning, communication, or action synchronization. The failure belongs to the team dynamic rather than to any single individual.

Choose if: A task requiring coordinated effort among multiple people failed due to inter-team communication or action synchronization breakdown.
→ Typical: Two separate teams on an offshore platform performing isolation and maintenance on the same line without knowledge of each other, transport and drilling teams working in the same zone during overlapping shifts in a mine, a commissioning team assuming a section completed by the testing team was ready when it was not at a power plant, a welder and a crane operator interpreting a lift signal differently on a construction site.

Not this if:

- Communication channel design and system are deficient → D2.1 (Horizontal communication inadequate)

- Shift handover protocol does not exist → D2.4 (Shift handover inadequate)

- SIMOPS planning is absent → D4.9 (Simultaneous operations)

- Cross-organizational communication mechanisms are missing → D2.3

- Coordination failure stems from absence of leadership → D1.2 or D1.8

- Psychological safety absence prevents surfacing of coordination errors → D1.11

B. IMMEDIATE CAUSES – CONDITIONS (Unsafe Conditions)

B1. PROTECTIVE & WARNING SYSTEM FAILURES

B1.1 Guards / Safety Devices Ineffective

The protective system is physically in place and operational; however, its design capacity is insufficient to prevent the hazard or limit the resulting harm. The device functions — but does not provide adequate protection.

Choose if: The guard is present and working, but its design capacity is inadequate against the actual hazard.
→ Typical: A barrier height on an oil/gas site that meets the regulatory minimum yet cannot arrest the actual fall energy, a dust containment barrier in a mine with insufficient structural capacity to withstand explosion overpressure, a rotating equipment guard at a power plant designed with gaps too large to contain ejected fragments, a scaffold safety net on a construction site too weak to support a potential fall load.

Not this if:

- The guard was deliberately removed or bypassed → A3.5 or A3.6

- The guard collapsed during the incident → B1.2 (Faulty guards/devices)

- The guard specification was incorrectly defined in risk assessment → D4.1 (Risk assessment inadequate)

- The design standard itself is deficient → D5.1 (Technical design errors)

- Deferred maintenance has degraded performance → D6.2 (Maintenance execution inadequate)

- The guard was not updated to reflect changed hazard conditions → D4.3 (MoC ineffective)

B1.2 Faulty Guards / Safety Devices

The guard was in place and appeared functional; however, at the moment of the incident it failed to perform its intended function — it collapsed, broke, or failed to activate.

Choose if: The guard experienced structural or functional failure at the moment of the incident.
→ Typical: A safety barrier dislodged by impact on an offshore platform, a protective cage that broke when struck by moving equipment in a mine, a safety cover that shattered under pressure at a power plant, a scaffold guardrail that collapsed under load on a construction site.

Not this if:

- The guard was deliberately bypassed → A3.5

- The guard design capacity is inadequate but it is still functioning → B1.1

- Deferred maintenance weakened the guard → D6.2 (Maintenance execution inadequate)

- Inspection program failed to detect prior deterioration → D6.3 (Inspection/calibration inadequate)

- Design flaws created the conditions for failure → D5.1

B1.3 Faulty PPE

The PPE type and protection class were correctly selected and used; however, the equipment itself failed due to a manufacturing defect, material fatigue, or physical damage.

Choose if: The correct PPE type was selected and used, but the equipment itself failed.
→ Typical: A chemical glove seam splitting and leaking on an oil site, a helmet inner cushion losing structural integrity upon rock impact in a mine, insulation on a high-voltage test glove cracking mid-use at a power plant, a fall arrest harness strap snapping at the moment of a fall on a construction site.

Not this if:

- The wrong PPE type was selected → A3.3 (Wrong use of PPE/methods) or A3.7 (PPE selection mismatch)

- The procurement process delivered a non-conforming product → D8.2 (Wrong product delivered)

- Storage conditions caused deterioration → D8.4 (Storage inadequacy)

- PPE inspection was not performed and deterioration went undetected → D6.3

- Counterfeit or substandard material is involved → D8.6 (Counterfeit/suspect material)

B1.4 Warning / Alarm Systems Ineffective

The system activated and issued an alarm; however, the timing, volume, visibility, or content of the warning was insufficient for the person to respond to the hazard in time.

Choose if: The system functioned but the warning was not early enough, loud enough, visible enough, or meaningful enough.
→ Typical: A gas alarm on an oil/gas site inaudible against ambient noise levels, an evacuation signal reaching operators too late in a mine, a warning light positioned outside the control panel operator's field of view at a power plant, a crane movement alarm on a construction site that did not allow workers sufficient reaction time.

Not this if:

- The system never activated at all → B1.5 (Warning/alarm systems faulty or missing)

- Design capacity of the system is fundamentally inadequate → D5.1 or D5.3

- Alarm habituation is a behavioral issue → A4.1 (Distraction/attention diverted)

- Maintenance deficiency degraded alarm performance → D6.2

B1.5 Warning / Alarm Systems Faulty or Missing

At the time of the incident, the warning or alarm system that should have activated either failed to do so, or no such system existed.

Choose if: An alarm should have sounded but did not, or the system was never installed.
→ Typical: A gas detector alarm failing to trigger due to sensor failure on an oil/gas site, a conveyor system fault alarm being out of service in a mine, a high-pressure alarm not triggering because the pressure transmitter was out of calibration at a power plant, no audible warning system installed for crane movements in a work zone on a construction site.

Not this if:

- The system activated but was insufficient → B1.4

- Inspection and testing program is the root failure → D6.3 (Inspection/calibration inadequate)

- The system was never designed into the facility → D5.1 or D5.7 (Technical risk analyses missing)

- MoC process failed to update the system after changes → D4.3

B1.6 Wrong PPE Specified

The system or procedure specified the wrong level or type of PPE from the outset. The problem is in the PPE specification process, not in the individual's selection or behavior.

Choose if: The PPE defined by the procedure or system is mismatched with the hazard type or severity.
→ Typical: A filter half-mask being set as the standard for IDLH H₂S concentrations on an oil/gas site, chemical-resistant gloves being specified for mechanical impact risks in a mine, an inadequate dielectric class glove being standardized for high-voltage testing at a power plant, low-strength anchor equipment being specified for elevated work on a construction site.

Not this if:

- An individual made the wrong PPE selection → A3.3 (Wrong use of PPE/methods)

- Risk assessment never identified this hazard → D4.1

- The PPE standard is outdated and not revised → D9.2 (SPP development ineffective)

- The procurement system delivered the wrong product → D8.1 or D8.2

- The person specifying PPE lacked sufficient expertise → C3.1

B2. TOOLS, EQUIPMENT & VEHICLES

B2.1 Equipment Failure (Unexpected)

Equipment failed unexpectedly due to internal defects or concealed material deterioration that the user could not have detected. There were no visible warning signs prior to the failure.

Choose if: There were no visible warning signs prior to failure and the user could not reasonably have anticipated it.
→ Typical: Sudden pipeline rupture due to internal corrosion on an oil site, unexpected fracture of a connector due to metal fatigue in a mine, sudden hydraulic line burst at a power plant, structural steel member failing due to an internal crack on a construction site.

Not this if:

- The defect was visible or known to the user → A2.3 (Using equipment with known defect)

- An inspection program should have detected the defect → D6.3 (Inspection/calibration inadequate)

- The maintenance strategy remained reactive → D6.1 (Maintenance strategy inadequate)

- Long-term material degradation is the underlying cause → B2.7 (Equipment integrity loss)

- Deferred maintenance was accepted and normalized → D6.6 (Deferred maintenance normalization)

B2.2 Inadequate Equipment Preparation

The equipment was not sufficiently cleaned, isolated, or otherwise prepared before maintenance or intervention began. Residual process contents, energy sources, or hazardous conditions remained present.

Choose if: The equipment was not taken through required preparation steps before work commenced.
→ Typical: A tank on an oil/gas site not drained and gas-freed before entry, residual pressure not released from a chemical reactor in a mine processing facility, a capacitor not discharged before electrical panel maintenance at a power plant, water not drained from a pipeline before cutting on a construction site.

Not this if:

- No preparation procedure exists → D9.1 (No SPP for task)

- The LOTO procedure is inadequate or incomplete → D4.5 (Energy isolation/LOTO ineffective)

- The permit to work system did not cover these steps → D4.4 (Work permit system failure)

- Responsibility for preparation was unclear → D2.3 or D2.4 (Communication/handover inadequate)

- The procedure was known but deliberately skipped → A1.1 or A1.3

B2.3 Tool Failure

The tool failed unexpectedly due to an internal defect or structural fracture while being used correctly. The problem lies in the tool's integrity, not in the user's behavior.

Choose if: The tool failed while being used in the correct manner.
→ Typical: A torque wrench handle snapping while operating within nominal capacity on an oil site, a drill bit casing unexpectedly ejecting during normal use in a mine, a power tool igniting due to an internal short circuit at a power plant, an angle grinder disc shattering during correct use on a construction site.

Not this if:

- The tool was being used incorrectly → A2.2 (Improper tool use)

- The defect was visible and known beforehand → A2.4 (Using tool with known defect)

- Required calibration or verification was not performed → B2.4 (Inadequate tool preparation)

- Inspection program failed → D6.3

- Counterfeit or substandard tooling is involved → D8.6

B2.4 Inadequate Tool Preparation

The tool was put into use without the required calibration, verification, or pre-use check being performed.

Choose if: Required preparation or verification was not completed before the tool was used.
→ Typical: Making a flange connection with an uncalibrated torque wrench on an oil site, taking gas measurements with an unverified detector in a mine, performing electrical measurements with an out-of-calibration voltmeter at a power plant, using a laser level instrument whose alignment was not verified on a construction site.

Not this if:

- The tool failed correctly while being used → B2.3 (Tool failure)

- No calibration program exists systemically → D6.3 (Inspection/calibration inadequate)

- No preparation procedure exists → D9.1

- The user was unaware of the preparation requirement → D3.1 or D3.2

B2.5 Vehicle Failure

A vehicle component failed unexpectedly due to an internal system failure the driver could not have detected.

Choose if: A vehicle component failed unexpectedly during operation.
→ Typical: Sudden loss of hydraulic braking on a field vehicle at an oil site, unexpected steering system failure on a mine machine, hydraulic lifting system collapse on a forklift during operation at a power plant, hydraulic cylinder fracture under load on a dump truck on a construction site.

Not this if:

- The defect was visible and known → A2.3

- Required periodic maintenance or pre-use checks were not performed → B2.6 (Inadequate vehicle preparation)

- Maintenance strategy remained reactive → D6.1

- Inspection program should have detected the fault → D6.3

B2.6 Inadequate Vehicle Preparation

The vehicle was deployed without completing required periodic maintenance or pre-use checks.

Choose if: Required pre-use checks or periodic maintenance were not performed before the vehicle was operated.
→ Typical: A field vehicle deployed on an oil site without completing daily checks, a mine machine put into operation without verifying set-point values, a forklift used at a power plant without checking fluid levels, a dump truck sent to site on a construction project without testing brake performance.

Not this if:

- No vehicle maintenance plan exists → D6.1 (Maintenance strategy inadequate)

- A maintenance plan exists but is being deferred → D6.6 (Deferred maintenance normalization)

- No monitoring or enforcement mechanism exists → D9.5

- The driver was unaware of pre-use check requirements → D3.1

B2.7 Equipment Integrity Loss (Corrosion / Erosion / Fatigue)

Long-term material degradation has progressed cumulatively, ultimately resulting in sudden failure or integrity loss. This category covers failures that arise not from a single event but from a time-extended deterioration process.

Choose if: Gradual material degradation led to the failure.
→ Typical: Pipeline rupture caused by internal corrosion on an oil/gas site, fracture of a load-bearing connector due to metal fatigue in a mine, loss of turbine blade integrity due to erosion at a power plant, structural integrity loss at the base of a steel pile due to ground-contact corrosion on a construction site.

Not this if:

- A degradation monitoring program (NDT, corrosion measurement, etc.) should have been detecting this → D6.3 (Inspection/calibration inadequate)

- Maintenance strategy did not account for this type of degradation → D6.1 (Maintenance strategy inadequate)

- Wrong material selection was made at the design stage → D5.1 (Technical design errors)

- Process conditions accelerating corrosion were not evaluated → D4.1 or D5.7

- Degradation was identified but intervention was deferred → D6.6

B3. UNEXPECTED EXPOSURE TO HAZARDOUS ENERGY / MATERIAL

B3.1 Fire or Explosion

An unexpected ignition or combustion event occurred.

Choose if: An uncontrolled ignition, fire, or explosion took place.
→ Typical: Vapour cloud formation and flash ignition during pipeline repair on an oil/gas site, combustible dust accumulation triggered by a spark in a mine, LPG leak near a control room flashing into a fire at a power plant, welder sparks igniting adjacent combustible material on a construction site.

Not this if:

- The fire risk was known and accepted → D4.1 (Risk assessment inadequate)

- A hot work permit was not obtained → D4.4 (Work permit system failure)

- JSA failed to identify this ignition scenario → D4.1

- Ignition source controls were left in place → A3.5 or D9.1

B3.2 Energized Electrical Systems

Work began on a system believed to be de-energized, but the system was still live.

Choose if: The system was assumed to be de-energized but was still live.
→ Typical: A circuit tagged "off" on an oil site remaining energized through a back-feed, a secondary transformer connection continuing to remain live after LOTO in a mine, a temporary power distribution board not fully isolated during maintenance at a power plant, a construction site panel not completely de-energized during work.

Not this if:

- The LOTO procedure is inadequate → D4.5 (Energy isolation/LOTO ineffective)

- The permit-to-work system did not cover this risk → D4.4

- The procedure was known but deliberately skipped → A1.1 or A3.5

- Energy isolation verification was not performed → D4.5

B3.3 Non-Electrical Energized Sources

Mechanical, hydraulic, pneumatic, or pressurized stored energy was released unexpectedly.

Choose if: Pressure, hydraulic, spring, or mechanical stored energy was released in an uncontrolled manner.
→ Typical: Residual pressure not vented before a line is opened on an oil/gas site, unexpected activation of a spring-actuated valve mechanism at a power plant, hydraulic accumulator back-pressure not permanently isolated on a mine machine, pre-stressed tendon releasing uncontrolled during cutting on a construction site.

Not this if:

- LOTO procedure does not cover this energy type → D4.5 (LOTO incomplete/ineffective)

- No procedure exists for this energy source → D9.1

- Risk assessment did not identify this energy type → D4.1

B3.4 Hazardous Chemicals

An unexpected leak, spill, or gas accumulation occurred.

Choose if: An uncontrolled chemical release took place.
→ Typical: H₂S accumulation from a flange leak on an oil/gas site, acid line rupture and chemical spill in a mine processing facility, transformer cooling oil leakage at a power plant, gas leak from cylinder storage on a construction site.

Not this if:

- Equipment integrity loss is the root cause → B2.7 (Corrosion/integrity loss)

- Asset integrity monitoring program failed → D6.7 or D6.3

- Risk assessment missed this release scenario → D4.1 or D5.7

B3.5 Combustible Dust / Dust Explosion

A dust accumulation ignited unexpectedly.

Choose if: Uncontrolled ignition of a dust accumulation resulted in an explosion or fire.
→ Typical: Coal dust accumulation triggered by a spark in a mine, silo explosion due to inadequate dust control at a flour facility, combustible aluminium dust accumulation creating a flammable atmosphere in a metalworking facility, wood shavings accumulation ignited by welding sparks on a construction site.

Not this if:

- Dust risk was not included in the assessment → D4.1 (Risk assessment inadequate)

- Housekeeping was systematically neglected → D6.6 or B4.7

- Dust control equipment failed → B1.x or D6.2

B3.6 Oxygen-Deficient Atmosphere

Unexpected oxygen depletion occurred in a confined space.

Choose if: Oxygen levels in a confined space dropped to a dangerous level unexpectedly.
→ Typical: Nitrogen injection displacing air in a tank on an oil/gas site, oxygen consumption through corrosion reactions in a mine tunnel, residual CO₂ concentration remaining after a fire suppression system discharge at a power plant, oxygen depletion due to fermentation in an enclosed excavation on a construction site.

Not this if:

- Atmospheric testing was not performed → D4.4 (Confined space permit system) or D4.1

- No confined space procedure exists → D9.1

- The control system did not identify this risk → D4.1

B3.7 Radiation

Unexpected or uncontrolled radiation exposure occurred.

Choose if: Unexpected or uncontrolled radiation exposure is involved.
→ Typical: Incorrect positioning of a shielded source during pipeline NDT on an oil/gas site, leakage from a damaged casing of a radioactive isotope measurement instrument at a power plant, uncontrolled use of a portable X-ray device during geological rock sampling in a mine.

Not this if:

- Source isolation was not carried out → D4.5 or D4.4

- Risk assessment missed this scenario → D4.1

- Equipment design defect caused the leakage → D5.1

B3.8 Extreme Temperatures

Unexpected hot or cold contact or extreme thermal stress occurred.

Choose if: Unexpected contact with a hot or cold surface, or extreme thermal ambient conditions, was involved.
→ Typical: Accidental contact with an uninsulated high-temperature pipe surface on an oil site, a steam burn from a suddenly opened steam cap at a power plant, molten metal splash during casting operations in a mine processing facility, contact with molten material during asphalt application on a construction site.

Not this if:

- Surface temperatures were known but not marked → A4.2 or B4.6

- JSA did not identify this thermal hazard → D4.1

- PPE selection did not account for this risk → B1.6 or A3.7

B3.9 Noise or Vibration

A brief but extremely high noise or vibration exposure contributed to the incident.

Choose if: Instantaneous extreme noise or vibration exposure caused injury or contributed to the incident.
→ Typical: A shockwave generated by sudden pressure relief valve activation on an oil site, unexpected extreme vibration during drilling in a mine, turbine resonance vibration loosening equipment connections at a power plant, sudden high-frequency vibration from a pneumatic hammer on a construction site.

Not this if:

- Chronic occupational exposure is the concern → health surveillance/C1.x categories

- Vibration control was not addressed in design → D5.3

- Noise measurement was not performed and not included in risk assessment → D4.1

B3.10 Storm or Natural Events

A weather condition or natural event directly triggered the incident.

Choose if: A weather event or natural disaster was the primary trigger of the incident.
→ Typical: A severe storm jeopardizing crane operations on an offshore platform, heavy rainfall destabilizing a slope in a mine, a lightning strike causing overvoltage that affected electrical systems at a power plant, high winds toppling scaffolding and endangering workers on a construction site.

Not this if:

- Weather conditions were known and work continued regardless → D4.1 or D1.4 (Production pressure)

- Emergency preparedness did not cover this scenario → D10.1 or D10.6

- Infrastructure design did not account for these natural risks → D5.1

B3.11 Stored Energy Unexpected Release

Energy stored in springs, capacitors, accumulators, or similar systems was released in an uncontrolled manner.

Choose if: Mechanical, electrical, or pressurized stored energy was released unexpectedly.
→ Typical: An actuator spring activating without isolation on an oil/gas site, a UPS system capacitor not discharged before maintenance at a power plant, a hydraulic accumulator back-feed not isolated on mine equipment, a pre-stressed concrete tendon releasing uncontrolled during cutting on a construction site.

Not this if:

- Energy isolation procedure does not cover this energy type → D4.5 (LOTO incomplete/ineffective)

- No procedure exists for this energy source → D9.1

- Risk assessment did not identify this energy type → D4.1

B3.12 Mechanical Hazards

Moving equipment, pinch points, or projectiles caused injury.

Choose if: Rotating or moving equipment, pinch points, or ejected objects created the hazard.
→ Typical: A rotating shaft catching loose clothing on an oil site, reaching into a conveyor nip point in a mine, turbine fan blade fragments ejecting at a power plant, metal fragments ejecting from a grinding disc shattering on a construction site.

Not this if:

- A guard is present but inadequate → B1.1

- Guard was deliberately removed → A3.5

- Risk assessment did not identify mechanical hazards → D4.1

- Equipment design did not minimize these hazards → D5.8 (Inherently safer design ignored)

B3.13 Uncontrolled Chemical Reaction (Runaway)

An exothermic reaction exceeded cooling capacity, or another loss of control caused the reaction to go beyond design limits.

Choose if: A process reaction exceeded its design boundaries.
→ Typical: Rapid temperature rise and pressure build-up due to cooling loss in a refinery reactor on an oil/gas site, an acid-base reaction exceeding neutralization capacity in a mine processing facility, an unexpected reaction caused by a chemical injection dosing fault at a power plant, an exothermic runaway in an epoxy curing system on a construction site.

Not this if:

- HAZOP/risk analysis missed this scenario → D5.7 (Technical risk analyses missing)

- Design did not include adequate safeguards → D5.1 or D5.8

- Process control system failed to detect the deviation → B1.5 or D6.3

- Emergency shutdown system did not activate → B1.2 or D6.3

B4. WORKSPACE LAYOUT & ENVIRONMENTAL CONDITIONS

B4.1 Sıkışık Düzeni / Kötü Yerleşim

Fiziksel düzen, güvenli erişimi, hareketi veya çalışmayı engellemiştir.

Seç eğer: Çalışma alanının fiziksel konfigürasyonu güvenli çalışmayı doğrudan engelliyorsa.
→ Tipik: Petrol-gaz sahasında boru hatlarının o kadar sıkışık olması ki kişisel koruyucu ekipmanıyla çalışmanın imkânsız hale gelmesi, maden ocağında makine hareketinin çalışanlarla aynı koridor içinde olması, enerji santralinde kritik valfin arkasına sıkıştığından müdahale edilememesi, inşaat sahasında tahliye yolunun malzeme yığınıyla fiilen kapanmış olması.

Bu değil eğer:

- Düzensizlik bireysel davranıştan kaynaklanıyorsa → A2.5 veya B4.7

- Tasarım hataları mevcutsa → D5.3 (Ergonomik tasarım hataları)

- Değişim yönetimi olmadan ekipman eklendiyse → D4.3 (MoC yetersizliği)

- Risk değerlendirmesi çalışma alanı planlamasını kaçırdıysa → D4.1

B4.2 Yetersiz Aydınlatma

Görev için yetersiz aydınlatma tehlikenin görülmesini veya göstergelerin okunmasını engelledi.

Seç eğer: Aydınlatma yetersizliği tehlikeyi görmeden veya doğru bilgiyi okumadan geçilmesine doğrudan katkıda bulunduysa.
→ Tipik: Petrol sahasında gece bakımı sırasında yerde biriken sıvının loş ortamda görülememesi, maden ocağında tünel aydınlatmasının yetersiz kalması nedeniyle yön işaretlerinin okunamaması, enerji santralinde yanmış lamba nedeniyle uyarı panelinin okunamaz hale gelmesi, inşaat sahasında gece vardiyasında açık çukurun fark edilememesi.

Bu değil eğer:

- Lamba arızası bakım sorunuysa → D6.2 (Bakım uygulaması yetersiz)

- Aydınlatma tasarımı baştan yetersizse → D5.3 veya D5.1

- Çalışan tehlikeyi görmesine rağmen fark etmediyse → A4.2

B4.3 Kaygan / Engebeli Yüzeyler

Zemin koşulları kayma veya takılmaya neden oldu.

Seç eğer: Yüzey durumu doğrudan kayma, takılma veya düşmeye yol açtıysa.
→ Tipik: Petrol sahasında sızdıran flanşın altında biriken yağlı su karışımının zemini kayganlaştırması, maden ocağında geçiş yolunda su birikintisi oluşması, enerji santralinde çatı bakımında yosun tutmuş yüzeyde kayma, inşaat sahasında kış koşullarında buz tutan rampa yüzeyi.

Bu değil eğer:

- Temizlik ihmalinden kaynaklanıyorsa → B4.7 (Kötü düzen/housekeeping)

- Zemin tasarımı baştan yetersizse → D5.3

- JSA bu tehlikeyi tanımlamadıysa → D4.1

- Bireysel fark etmeme davranışıysa → A4.2

B4.4 Yüksekte Yetersiz Koruma / Düşme Tehlikesi

Düşme koruması mevcut değil veya yetersiz.

Seç eğer: Yüksekte çalışma için düşme koruması eksik veya yetersizse.
→ Tipik: Petrol platformunda açık bırakılan güverte deliğinde korkuluk olmaması, maden ocağında şev kenarına yakın çalışma alanında güvenlik halatı bağlantı noktasının bulunmaması, enerji santralinde çatı bakımında tamamlanmamış korkuluk sistemi, inşaat sahasında iskele platformunda yeterli bariyer yüksekliğinin sağlanmaması.

Bu değil eğer:

- Güvenlik halatı mevcut ama kullanılmadıysa → A3.2

- Düşme koruma ekipmanı KKD olarak mevcut değilse → A3.4

- Tasarım hataları varsa → D5.1 veya D5.3

- Risk değerlendirmesi bu bölgeyi kaçırdıysa → D4.1

- Kontroller planlandı ama sahaya uygulanmadıysa → D4.2

B4.5 Yetersiz Havalandırma

Yetersiz hava sirkülasyonu veya duman/gaz/toz ekstraksiyonu tehlikeli konsantrasyonlar oluşturdu.

Seç eğer: Hava sirkülasyonu yetersizliği doğrudan tehlikeli atmosfer koşulu yarattıysa.
→ Tipik: Petrol-gaz sahasında kapalı bölmede kaynak işlemi sırasında duman birikmesi, maden tünelinde motorlu ekipman egzoz gazının yeterince tahliye edilememesi, enerji santralinde trafo odasında elektrikli arıza sonrası oluşan ozon ve yanık koku konsantrasyonu, inşaat sahasında kapalı alanda epoksi uygulama sırasında çözücü buharlarının birikmesi.

Bu değil eğer:

- Havalandırma tasarımı baştan yetersizse → D5.1 veya D5.3

- Havalandırma ekipmanı arıza nedeniyle çalışmıyorsa → D6.2 (Bakım yetersiz)

- Kapalı alan izin sistemi ölçüm gerektirmiyorsa → D4.4

- Risk değerlendirmesi bu konsantrasyon riskini tanımlamadıysa → D4.1

B4.6 Yetersiz veya Okunamaz Etiketleme / İşaretleme

Etiketler eksik, soluk veya hatalıdır.

Seç eğer: Etiket veya işaret eksikliği, silinmişliği ya da yanlışlığı olayın oluşmasına doğrudan katkıda bulunduysa.
→ Tipik: Petrol sahasında akışkan türünü belirtmeyen boru hattı etiketinin neden olduğu yanlış valf açılması, maden ocağında hasar görmüş tehlike işaretinin güvensiz bölgeyi belirtememesi, enerji santralinde solmuş devre kesici etiketinin karıştırılarak yanlış birimin devre dışı bırakılması, inşaat sahasında kimyasal varil etiketinin kazıma nedeniyle okunamaz hale gelmesi.

Bu değil eğer:

- Etiketleme prosedürü baştan yoksa → D9.1

- Bakım programı etiketi güncellemiyorsa → D6.2

- MoC süreci etiketleri güncellemediyse → D4.3

- Tedarik sürecinde malzeme etiketsiz geldiyse → D8.5 (Etiketleme yetersizliği)

B4.7 Kötü Düzen / Housekeeping

Çalışma alanında dağınıklık, döküntü veya engelleyici nesneler mevcut.

Seç eğer: Çalışma alanındaki düzensizlik, takılma, çarpma veya erişim engeli olayına doğrudan katkıda bulunduysa.
→ Tipik: Petrol sahasında boru geçiş koridoruna bırakılan alet çantası ve malzemelerin takılmaya yol açması, maden ocağındaki hurda metal yığınının tahliye yolunu daraltması, enerji santralinde kontrol odası zemin kablolarının düzensiz bırakılması, inşaat sahasında kesim artıklarının çalışma platformunda birikmesi.

Bu değil eğer:

- Kötü düzen tasarımdan kaynaklanıyorsa → B4.1 (Sıkışık düzen)

- Denetim yokluğu bu koşulu sürdürüyorsa → D1.2 (Yetersiz denetim)

- Temizlik prosedürü mevcut değilse → D9.1

- Bakım ertelemesi bu birikimi yarattıysa → D6.6

B4.8 Çalışma Alanı Tasarımı Hata Olasılığını Artırıyor

Tasarım, insan hatasını sistematik biçimde kolaylaştırmaktadır.

Seç eğer: Çalışma alanı veya ekipman düzeni, ortalama bir kişinin yanlış işlem yapmasını sistematik biçimde kolaylaştırıyorsa.
→ Tipik: Petrol-gaz sahasında birbirine çok benzeyen iki valfin renk veya konum farkı olmaksızın yan yana yerleştirilmesi, maden proses kontrol panelinde farklı işlevli düğmelerin aynı görsel yapıda tasarlanmış olması, enerji santralinde sık kullanılan kontrol düğmesiyle acil durdurma butonunun aynı panelde bitişik konumlandırılması, inşaat sahasında beton karışım istasyonunda kilogram ve litre birimlerinin aynı ekranda birbirini izleyerek görüntülenmesi.

Bu değil eğer:

- Bireysel dikkat dağınıklığı veya dikkatsizlik söz konusuysa → A4.1 veya A4.5

- Ergonomik tasarım kök neden ise → D5.3 (Ergonomik tasarım hataları)

- Etiketleme yetersizliği karışıklığa yol açıyorsa → B4.6

- Prosedürün görev adımlarını yeterince açıklamaması sorun ise → D9.2

B4.9 Çalışma Alanı Göstergeleri Yetersiz

Göstergeler, ekranlar veya etiketler yanlış, eksik veya gecikmiş bilgi sağladı.

Seç eğer: Göstergeler, ölçüm cihazları veya ekranlar yanlış, eksik veya gecikmiş bilgi sağladıysa.
→ Tipik: Petrol sahasında gerçek hat basıncını doğru göstermeyen arızalı basınç transmitterinin yanlış kararı tetiklemesi, maden kontrol odasında donmuş ekran görüntüsünün işlemcinin gerçek durumunu yansıtmaması, enerji santralinde gerilim değerini hatalı gösteren arızalı multimetrenin yanlış devre dışı bırakma kararına yol açması, inşaat sahasında kümülatif yük hesabını yanlış yapan elektronik kaldırma ölçer.

Bu değil eğer:

- Tasarım hatası bilgiyi sistematik biçimde yanıltıcı kılıyorsa → D5.3 veya D5.1

- Bakım programı arızayı gidermemişse → D6.2

- Kalibrasyon yapılmadığı için yanlış okuma oluşuyorsa → D6.3 (Muayene/kalibrasyon yetersiz)

- Kişi doğru bilgiyi görmesine rağmen yanlış yorumladıysa → A4.5 veya A2.8

B4.10 Hava Koşulları (Yağmur / Rüzgar / Buz)

Açık hava çalışması olumsuz hava koşullarından etkilendi.

Seç eğer: Hava koşulları çalışma ortamını doğrudan tehlikeli hale getirdiyse.
→ Tipik: Petrol platformunda güçlü rüzgarda açık güverte bakımı, maden ocağı üstü şev alanında yoğun yağış sonrası zemin kayması, enerji santralinde buz tutan çatı yüzeyinde bakım çalışması, inşaat sahasında soğuk havada asfalt uygulamasında buz oluşumu.

Bu değil eğer:

- Hava koşulları biliniyordu ve çalışmaya devam edildi → D4.1 (Risk değerlendirmesi) veya D1.4 (Üretim baskısı)

- JSA hava koşulları değerlendirmesini içermiyorsa → D4.1

- Doğal felaket düzeyinde bir olaysa → B3.10 (Fırtına veya doğal olaylar)

C. SYSTEMIC ROOT CAUSES – PERSONAL FACTORS

C1. PHYSICAL CAPACITY & HEALTH

C1.1 Sensory Disorders

The person's permanent sensory limitations — such as impaired vision, hearing loss, or color blindness — directly prevented the task from being performed safely. This is not a temporary condition; it is a pre-existing, permanent limitation.

Choose if: A permanent sensory limitation directly prevents the perceptual capacity required for safe task execution.
→ Typical: An operator on an oil/gas site misreading a pressure gauge due to uncorrected visual impairment, hearing loss preventing a mine worker from detecting an evacuation alarm, color blindness causing a power plant technician to fail to distinguish warning light colors, visual impairment making it difficult for a construction site worker to track crane lift signals.

Not this if:

- The limitation is temporary or caused by fatigue → C1.4 (Fatigue)

- The key question is why the person was assigned to this task with a known limitation → D3.5 (Critical competency gaps) or C3.1

- Warning system design does not account for sensory limitations → D5.3 (Ergonomic design errors)

- Recruitment or periodic health screening system failed to identify the limitation → organizational procedure issue

C1.2 Physical Limitations

The person's height, strength, reach, or physical capacity does not meet the demands of the task. This is a permanent physical mismatch.

Choose if: A permanent physical limitation directly prevents the physical capacity required for safe task execution.
→ Typical: An operator on an oil/gas site forced into an unstable position because they lack the height to reach a control, a mine worker assigned a manual handling task that exceeds their carrying capacity, a person physically unsuitable for a confined-passage task at a power plant being assigned to that role, a construction worker with an arm-hand disorder being continued on vibrating equipment tasks.

Not this if:

- The task can be redesigned to eliminate the limitation → D5.3 (Ergonomic design errors)

- The task assignment system failed to prevent this mismatch → C3.1 (Required skill assessment inadequate)

- The limitation was known but the task assignment was not changed → organizational procedure issue

C1.3 Medical Conditions or Illness

A known or unknown medical condition, or a sudden health event, directly affected the person's performance during the task.

Choose if: A medical condition or health event can be clearly established as having directly impaired the person's performance during the task.
→ Typical: A sudden blood sugar drop due to diabetes causing confusion during a critical operation on an oil site, an undisclosed cardiac condition resulting in sudden loss of consciousness in a mine, an epilepsy patient experiencing a seizure and losing control of a panel at a power plant, a prescription medication side effect impairing attention and coordination on a construction site.

Not this if:

- The condition was known and the question is why the person was assigned to this task → C3.1 or D3.5

- Periodic health screening should have identified the condition → organizational/OHS management system issue

- Impairment from medication is the result of deliberate substance misuse → C1.5

C1.4 Fatigue (Acute or Chronic)

Insufficient rest, extended working hours, or excessive overtime has significantly degraded the person's physical and cognitive performance.

Choose if: Insufficient rest or excessive working hours visibly degraded performance and contributed to the incident.
→ Typical: An operator on an oil/gas site in their 16th hour of work missing a critical parameter change, a mine worker on their third consecutive night shift experiencing coordination loss, a technician performing a commissioning task after a 12-hour emergency response at a power plant, a supervisor moving between multiple projects on a construction site experiencing attention deficits.

Not this if:

- Fatigue stems from organizational scheduling → D1.4 (Production pressure) or shift scheduling

- Shift rotation disrupts the biological cycle → C1.6 (Shift schedule circadian disruption)

- The workload is a design error that exceeds one person's capacity → D5.3 or A4.7

C1.5 Drugs, Alcohol, or Medication

Substance use impaired the person's performance, judgment capacity, or motor skills.

Choose if: Substance impairment can be confirmed or reasonably inferred as having affected performance.
→ Typical: An employee under the influence of alcohol making incorrect process control decisions on an oil site, a prescribed medication slowing reaction time in a mine, a night-shift technician taking sleep medication experiencing attention loss at a power plant, pain medication impairing coordination on a construction site.

Not this if:

- Deliberate misuse and policy violation are the primary concern → both an individual issue and D1.3 (accountability)

- Medication side effects were unknown and the person was not informed → D3.1 or procedure issue

- Testing and screening system failed → organizational/policy issue

C1.6 Shift Schedule Circadian Disruption

Night shifts or rapid rotation created a mismatch with the biological cycle, reducing the person's alertness level and cognitive capacity.

Choose if: The shift rotation pattern or night work disrupted the biological cycle and this contributed to the incident.
→ Typical: An operator on an offshore platform switching to a night shift through rapid rotation experiencing alertness reduction between 03:00–05:00, a mine worker's sleep quality degraded by 12-hour day-night rotation, a power plant technician returning to night shifts every 4 weeks with the cycle unable to adapt before the next change, a manager on a cross-border project experiencing impaired decision-making capacity due to time zone transitions.

Not this if:

- General fatigue is the concern → C1.4 (Fatigue)

- Shift scheduling is an organizational decision → D1.4 or HR/planning system

- Sleep disorder is a medical condition → C1.3

C2. COGNITIVE & MENTAL ABILITY

C2.1 Memory or Attention Limitations

The task required extensive memorization or sustained concentration without written support, and the person was unable to meet this cognitive demand.

Choose if: The task genuinely required written support that did not exist, and as a result the person's memory or attention capacity was exceeded. Select ONLY when the person's capacity is the primary issue.
→ Typical: An operator on an oil/gas site required to execute a 30-step commissioning procedure from memory without a checklist, a team leader in a mine trying to retain information from multiple shifts without a written logging system, a power plant technician expected to recall a rarely applied emergency procedure without a reference document.

Not this if:

- A procedure or checklist should have existed but did not → D9.1 (No SPP for task) — this is most often the true root cause

- Fatigue has reduced memory capacity → C1.4

- Task design creates demands that exceed memory → A4.7 or D5.3

- Training deficiency has created a knowledge gap → D3.2 or D3.4

C2.2 Poor Coordination or Reaction Time

The motor skill, coordination, or reaction speed demands of the task exceeded the person's physical capacity.

Choose if: The task required a specific level of motor skill or reaction speed and the person's capacity did not meet that requirement.
→ Typical: A worker with insufficient motor coordination assigned to a task requiring precise two-handed valve control on an oil site, an older operator assigned to a role requiring rapid response to sudden mechanical failure in a mine, a person with a tremor condition performing fine connection work at a power plant, a person unsuitable for precision welding requiring hand coordination being assigned to that task on a construction site.

Not this if:

- The task can be redesigned to eliminate this requirement → D5.3

- The task assignment process failed to prevent this mismatch → C3.1

- Coordination loss is temporary → C1.4 or C1.5

C2.3 Poor Mechanical / System Comprehension

The person was unable to grasp the fundamental operating principles of the system or equipment they were working with. This is not a rule knowledge gap — it is a deep understanding gap.

Choose if: The person cannot understand at a fundamental level how the system works, and this was determinative in the incident.
→ Typical: An operator on an oil/gas site unable to grasp pressure-volume relationships and therefore unable to anticipate process changes, a mine technician who does not understand electrical circuit logic and cannot see the back-feed risk, a maintenance worker at a power plant who does not know hydraulic system principles and incorrectly determines isolation sequence, a construction assembly worker who cannot grasp structural load distribution and incorrectly positions supports.

Not this if:

- Training design did not build conceptual understanding → D3.2 (Training design ineffective)

- Competency assessment failed to detect this gap → C3.1

- Task assignment system overlooked this mismatch → D3.5

C2.4 Inadequate Judgment or Decision-Making

An ambiguous or unresolved situation required the person to use their own assessment, and that assessment proved inadequate.

Choose if: No clear procedure covered the situation, the person was genuinely required to use their own judgment in the field, and that judgment was inadequate. Select ONLY in genuine ambiguity situations.
→ Typical: An operator on an oil/gas site making incorrect prioritization decisions when facing an unexpected process anomaly not covered by any procedure, a mine foreman making the wrong decision when rock conditions change and the situation falls outside existing procedures, a power plant technician making an incorrect intervention decision during a new equipment failure when the available procedure does not cover the situation.

Not this if:

- A clear procedure exists and the person did not follow it → A1.1 (Individual rule violation)

- The procedure covers the situation but is ambiguous → D9.2 (SPP development ineffective)

- Decision-making training was not provided → D3.2

- Lack of experience affected judgment → C3.2 or C3.4

C2.5 Performance-Affecting Emotional State

Stress, fear, phobia, anxiety, or a traumatic trigger directly impaired the person's ability to make safe decisions or take appropriate action during the task.

Choose if: An emotional or psychological state directly prevented the person from making safe decisions or acting appropriately during the task.
→ Typical: A worker with a height phobia on an oil site resorting to an unsafe movement in a panic state, a mine worker who previously experienced an accident responding with a PTSD reaction in a similar environment, a person experiencing intense panic during an emergency at a power plant being unable to execute the procedure, a manager under severe work pressure on a construction site demonstrating elevated risk tolerance.

Not this if:

- The condition was known and the question is why the person was assigned → C3.1 or organizational assignment issue

- Psychological safety culture prevents these issues from being disclosed → D1.11

- Stress originates from organizational pressure → D1.4 (Production pressure)

C2.6 Lack of Task-Specific Mental Models

Standard training was provided; however, the person was unable to build a functional mental framework from this knowledge that would allow them to predict system behavior.

Choose if: The person received training, knows the rules, but cannot mentally model the dynamic behavior of the system, and this was determinative in the incident.
→ Typical: An operator on an oil/gas site who has memorized process flow diagrams but cannot anticipate how a pressure change propagates along the line, a mine worker who knows safety rules but cannot internalize why a ceiling load change can create an immediate hazard, a power plant technician who can read a circuit diagram but cannot visualize where back-feed current might come from.

Not this if:

- Training was never provided → D3.1

- Training design was built only on rule memorization rather than conceptual understanding → D3.2 (Training design ineffective) — this is most often the true root cause

- Individual comprehension difficulty is systemic → C2.3

C3. SKILL LEVEL, COMPETENCE & BEHAVIORAL CONDITIONING

C3.1 Required Skill Assessment Inadequate

The person was assigned to the task under the assumption of competence; however, they did not in fact possess the required competency. The problem lies not in the individual but in the competency verification process.

Choose if: The person was assigned without adequately verifying whether they possessed the competency required for that specific task.
→ Typical: An inexperienced crane operator assigned to a critical lift on an oil/gas site, a person who has not yet obtained explosive storage authorization carrying out that task in a mine, a technician without a high-voltage test certificate performing that work at a power plant, a person without site management experience being assigned to manage a complex SIMOPS operation on a construction site.

Not this if:

- Assessment was conducted but training design was inadequate → D3.2

- No competency matrix or assessment system exists at the organizational level → D3.5 (Critical competency gaps)

- The task changed but competency was not updated → C3.7

C3.2 Insufficient Skill Practice

Theoretical knowledge is adequate; however, the level of field practice or hands-on experience required by the task has not been acquired.

Choose if: The person received training and has theoretical knowledge, but the skill has not been translated into real performance due to insufficient field application.
→ Typical: A technician on an oil/gas site who passed the LOTO procedure in a classroom but has never independently applied it in the field, a new mine worker who has never participated in an emergency evacuation drill, a control room operator at a power plant who transitions to a live system without having practiced on a simulator, a construction worker who passed a theory exam but begins working at height having never actually practiced it.

Not this if:

- Training was never provided → D3.1

- Field coaching or supervised practice was not planned → C3.3 (Skill coaching not done)

- Skill deteriorated over time and no refresher was provided → C3.4

C3.3 Skill Coaching Not Done

The person was left to perform the task without receiving field training, mentoring, or supervised practice.

Choose if: The person completed theoretical training but did not receive hands-on coaching support under the supervision of an experienced person in the field.
→ Typical: A new hire on an oil/gas site who completed the training period being left to perform a critical task without mentor assignment, a mine operator sent into the field without an experienced foreman accompanying them after classroom training, a person at a power plant beginning work on a complex system commissioning task without the support of an experienced technician, a new equipment operator on a construction site transitioning to real operations without supervised practice.

Not this if:

- Field practice was conducted but was insufficient → C3.2

- The training system does not plan a coaching phase → D3.2 (Training design ineffective)

- Supervision absence is at the organizational level → D1.2 (Inadequate supervision)

C3.4 Skill Rarely Applied or Degraded

The skill has atrophied after an extended gap in use and performance has fallen below a safe level.

Choose if: The person once possessed this skill; however, competency has significantly degraded due to prolonged non-use.
→ Typical: An operator on an oil/gas site who has not applied an emergency shutdown procedure for years hesitating at a critical moment, a specialized equipment maintenance task in a mine being performed incorrectly when carried out for the first time in several years, a power plant technician called in for an annual test being unable to recall the procedure after a long interval, a rarely used specialized welding technique being incorrectly applied when recalled after a year on a construction site.

Not this if:

- Refresher training was never planned → D3.4 (Knowledge forgotten) — organizational issue

- The training system has not established a practice cycle for rare tasks → D3.2

- The competency matrix does not track this skill → D3.5

C3.5 Wrong Behavior Rewarded

Unsafe behavior resulted in time savings, increased production, or management approval, thereby reinforcing the behavior.

Choose if: It can be clearly established that unsafe behavior produced a positive outcome (speed, praise, reward) and was therefore repeated.
→ Typical: A worker on an oil/gas site who skipped a LOTO step receiving appreciation from their supervisor for speed, a mine team that bypassed a safety protocol winning a production bonus, a person at a power plant who finished work early by shortening a procedure receiving a "good job" message, a construction site foreman who quickly moved through safety steps receiving a promotion recommendation from management.

Not this if:

- Leadership is not actively encouraging these behaviors but is ignoring them → C3.6 (Correct behavior not rewarded)

- Management is knowingly tolerating the behavior → D1.9 (Management tolerance of known deviations)

- Behavioral reinforcement has become a systemic culture → D1.5 or D1.10

C3.6 Correct Behavior Not Rewarded

Safe behavior has been systematically ignored, devalued, or labelled as a slowdown.

Choose if: It can be clearly established that safe behavior was consistently perceived negatively or never recognized.
→ Typical: A worker on an oil/gas site who complies with safety procedures being consistently criticized for "slowing work down," a mine worker who reports a hazard being labelled a "troublemaker" by the team, a power plant technician who uses an emergency stop being made to justify why it was necessary, a construction site foreman who requests time for a risk assessment being pressured by superiors.

Not this if:

- Unsafe behavior was directly rewarded → C3.5 (Wrong behavior rewarded)

- Safety leadership is absent → D1.8 or D1.10

- Psychological safety culture normalizes this behavior → D1.11

C3.7 Task Change Competence Gap

The person was transferred to a new role or task without a competency verification being conducted prior to the change.

Choose if: The person was rotated to a different role or task and the required competency assessment for this transition was not carried out.
→ Typical: A production operator on an oil/gas site transferred to a maintenance role without relevant training, a surface team member in a mine assigned to underground operations without an orientation and familiarization program, a control room operator at a power plant transitioning from a legacy system to a new digital control system beginning work without a competency transition program, an engineer or craftsperson from a different discipline on a construction site being brought in for a critical phase without preparation.

Not this if:

- The initial task assignment assessment was inadequate → C3.1 (Required skill assessment inadequate)

- Organizational workforce planning is creating systemic gaps → D3.5

- The MoC process failed to update the competency requirements resulting from the task change → D4.3

D. SYSTEMIC ROOT CAUSES – ORGANIZATIONAL FACTORS

D1. LEADERSHIP, SUPERVISION & SAFETY CULTURE

D1.1 Weak Leadership Commitment

Safety is systematically deprioritized by senior management in budget allocation, resource decisions, and strategic planning. This is not the behavior of an individual manager — it is a concrete indicator of the value the organization places on safety.

Choose if: It can be clearly established that safety resources, budget, or strategic priority are consistently sidelined by senior management decisions.
→ Typical: A safety investment project on an oil/gas site being repeatedly deferred and removed from the budget, training budget being the first line item cut every year in a mine, critical equipment renewal requests being rejected on cost grounds at a power plant, safety performance never appearing on the board agenda in a construction company.

Not this if:

- A field-level individual manager has a supervision problem → D1.2 (Inadequate supervision)

- Resource absence originates from the procurement system → D8.x

- Production pressure is a systemic cultural issue → evaluate together with D1.4

- Leadership visibility absence is at field level → D1.8

D1.2 Inadequate Supervision

Frontline managers and supervisors are not sufficiently present and effective at field level. Hazards go unnoticed, violations go unseen, and no intervention occurs.

Choose if: First-line managers are not actively present in the field and this absence directly contributed to the incident.
→ Typical: A supervisor on an oil/gas site responding only reactively without awareness of daily hazards, a mine foreman never conducting in-person checks of tunnel work, a power plant maintenance supervisor attempting to manage critical commissioning activities remotely, a construction site manager unable to adequately supervise any single area while managing multiple sites simultaneously.

Not this if:

- The manager saw the violation and ignored it → D1.9 (Management tolerance of known deviations)

- The manager personally committed the violation → A1.3

- Systematic absence of leadership visibility is involved → D1.8

- Supervision mechanism is absent at the organizational level → D9.5

D1.3 Lack of Accountability

Deviations have been identified or reported; however, no consequence, corrective action, or follow-up mechanism has been triggered.

Choose if: Violations or deviations were known yet no consequence or corrective action was applied.
→ Typical: Repeated procedure violations on an oil/gas site being reported but no disciplinary or corrective process being initiated, audit findings in a mine being presented as a report to senior management but never implemented, the same safety violation being observed multiple times at a power plant with no feedback given to the person involved, an action plan prepared after an incident on a construction site never being followed up.

Not this if:

- The manager was genuinely unaware of the violation → D1.2 (Inadequate supervision)

- Cultural normalization is involved → D1.5

- Management is knowingly tolerating the behavior → D1.9

- Reporting mechanism does not exist → D1.7

D1.4 Production Pressure

Output targets are overtly or implicitly placed ahead of safety requirements. This is not individual pressure — it is a systemic issue concerning how the organization balances safety and production.

Choose if: The message that production targets take priority over safety is being communicated to workers, explicitly or implicitly.
→ Typical: A manager on an oil/gas site sending the message "don't miss the delivery, do whatever it takes," safety checks being cut short at end of shift to meet tonnage targets in a mine, commissioning tests being partially skipped under customer pressure at a power plant, critical control steps being compressed to avoid delay penalties on a construction site.

Not this if:

- An individual rule violation is involved → A1.1

- Time pressure is an individual, in-the-moment personal choice → A4.8

- Leadership commitment issue is at senior management level → D1.1

- Absence of psychological safety is preventing this pressure from being reported → D1.11

- If production/schedule pressure led management to consciously accept a contractor that failed prequalification → D7.3

D1.5 Normalization of Deviance ⚠️ USE WITH GREAT CARE

A specific violation or deviation has become culturally accepted through a wide-scale, long-running, multi-person process.

⚠️ ALL 5 OF THE FOLLOWING CRITERIA MUST BE MET:

- The same violation was repeated multiple times (not once or twice)

- It is carried out by more than one person (not individual)

- It has continued for an extended period (weeks/months, not days)

- No incident has yet occurred as a result (if an incident has occurred → immediate cause categories apply)

- Gradual cultural acceptance has formed (not a sudden decision)

Choose if: Long-term, widespread, and systematic normalization of a specific violation is present.
→ Typical: An entire department on an oil/gas site skipping LOTO for years, with this becoming a "way of working" that no one questions, multiple crews in a mine committing the same procedure violation for months with management treating it as normal.

Not this if:

- It is occurring for the first time → A1.1 (Individual rule violation)

- Management is aware → D1.9 (Management tolerance of known deviations)

- LOTO was skipped once or twice → D9.5 or D1.9

- No procedure exists → D9.1

- Maintenance deferral is involved → use D6.6, not D1.5

- The manager is unaware → D1.2

- Accountability is unclear → D1.3

D1.6 Stop Work Authority Ineffective

Stop work authority exists procedurally; however, workers are unable to exercise it because they feel unsafe doing so.

Choose if: The authority exists on paper but workers cannot realistically exercise it in practice.
→ Typical: A worker on an oil/gas site recognizing an unsafe condition but avoiding stopping work out of fear of retaliation, a mine worker who reports an unsafe condition being labelled "overreacting" or a "troublemaker," a person at a power plant who stops work receiving a negative note in subsequent evaluations, subcontractor workers feeling unable to exercise this authority against a main contractor's supervisor on a construction site.

Not this if:

- Psychological safety culture is involved → evaluate together with D1.11

- The authority has never been procedurally defined → D9.1

- Accountability mechanism does not exist → D1.3

D1.7 Weak Reporting / Learning Culture

Near-misses and safety concerns are not being reported, or when reported, they do not result in any learning or corrective action process.

Choose if: The system is failing to capture near-misses and concerns and convert them into a learning process.
→ Typical: Workers on an oil/gas site not reporting because "reporting changes nothing," near-miss reports in a mine being filed and never worked through, an incident reporting system at a power plant being perceived as intimidating and bureaucratic and therefore not used, near-misses on a construction site being passed over through verbal communication only.

Not this if:

- Absence of psychological safety is preventing reporting → evaluate together with D1.11

- Accountability issue is causing reports to go unaddressed → D1.3

- Incident investigation quality is a systemic issue → separate organizational capacity problem

D1.8 Insufficient Visible Leadership

Managers are not present in the field, safety dialogues are not occurring, and leadership field engagement is absent.

Choose if: Manager field visits, safety observation tours, or safety conversations are systematically absent.
→ Typical: A facility manager on an oil/gas site having not conducted a field tour for months, a mine foreman never personally observing hazardous conditions inside the tunnel, power plant maintenance supervisors managing critical work remotely without going to the field, a construction project manager working only in meeting rooms.

Not this if:

- The manager was aware but did not act → D1.9

- Leadership commitment is a senior management level issue → D1.1

- Safe leadership behaviors are not being reinforced → D1.10

D1.9 Management Tolerance of Known Deviations

The manager is aware that a violation or deviation is occurring; however, they are not intervening, correcting, or reporting it.

Choose if: It can be clearly established that a specific manager was aware of the violation and consciously chose to ignore it.
→ Typical: A supervisor on an oil/gas site staying silent about a daily bypassed safety check to maintain team speed, a mine foreman aware of a bypassed safety device but not intervening to avoid production disruption, maintenance supervisors at a power plant routinely observing non-standard temporary repairs without recording them, a construction site manager knowing PPE is not being used but dismissing it as "a small job."

Not this if:

- The manager is genuinely unaware of the violation → D1.2 (Inadequate supervision)

- Cultural normalization (5 criteria) is involved → D1.5

- The manager personally ordered the violation → A1.3

D1.10 Leadership Not Reinforcing Safe Behaviors

Safe behaviors are never recognized by managers, never discussed during observation tours, and never reflected in reward systems.

Choose if: It can be clearly established that safe behaviors are systematically ignored or unrecognized by leadership.
→ Typical: Safety observation tours on an oil/gas site noting only negative findings and never recording good practices, a mine worker who reports a hazard receiving no acknowledgment, safety behaviors never appearing in performance evaluations at a power plant, a worker who demonstrates safety leadership on a construction site receiving the same evaluation score as all others.

Not this if:

- Unsafe behavior is being actively rewarded → C3.5

- Psychological safety culture is a broader issue → D1.11

- Leadership visibility absence is involved → D1.8

D1.11 Absence of Psychological Safety

Workers feel that speaking up, admitting mistakes, or reporting unsafe conditions will result in blame, exclusion, or retaliation.

Choose if: It can be clearly established that workers are afraid to speak up, ask questions, or report concerns.
→ Typical: A worker on an oil/gas site who asks a question about a procedure being met with "don't ask so many questions," a mine worker who reports an unsafe condition being placed on an undesirable night shift in the next schedule, a power plant technician who admits a mistake receiving a disciplinary penalty, a subcontractor worker on a construction site being unable to report a dangerous working condition out of fear of losing their job.

Not this if:

- Reporting culture is a more specific issue → evaluate together with D1.7

- Stop work authority cannot be exercised → evaluate together with D1.6

- Accountability mechanism is creating a climate of fear → D1.3

D2. COMMUNICATION & INFORMATION MANAGEMENT

D2.1 Horizontal Communication Inadequate

The flow of information between workers, teams, or units at the same organizational level is insufficient.

Choose if: It can be clearly established that critical information was not shared between workers or teams at the same level or in the same area.
→ Typical: A day shift on an oil/gas site not informing the night shift about a leak that developed on the line, a mine crew not sharing a rock crack they detected with the adjacent tunnel crew, a team at a power plant dismissing a vibration anomaly they noticed without communicating with another team working in the same area, a concrete crew on a construction site not sharing a measurement deviation with the formwork crew.

Not this if:

- Information is being lost between different hierarchical levels → D2.2 (Vertical communication inadequate)

- Shift handover protocol is the primary issue → D2.4

- Communication breakdown exists between different organizations → D2.3

D2.2 Vertical Communication Inadequate

The flow of information between management and field workers — downward or upward — is insufficient.

Choose if: Critical information was not transmitted or did not reach its destination between management levels and the field.
→ Typical: A new process hazard identified by management on an oil/gas site not being communicated to field operators, an equipment deterioration reported by a mine worker never reaching the management agenda, a risk identified by senior engineers at a power plant not being passed on to field technicians, a design change on a construction site not being communicated to field craftspeople in time.

Not this if:

- Communication breakdown exists between workers at the same level → D2.1

- Information was transmitted but misunderstood → D2.7 (Information misunderstood)

- Technical documentation is not current → evaluate together with D2.8

D2.3 Inter-Organizational Communication Inadequate

Coordination and information sharing between different organizational units — such as maintenance, operations, engineering, or contractors — is insufficient.

Choose if: An information gap between different departments, units, or organizations contributed to the incident.
→ Typical: A maintenance crew on an oil/gas site beginning work on a line without notifying the operations department, a contractor drilling crew in a mine being unaware of the main contractor's blasting schedule, an engineering-approved design change at a power plant not being communicated to the maintenance team, different subcontractors on a construction site working in the same area without coordination.

Not this if:

- Vertical communication issue within the same organization → D2.2

- SIMOPS planning is inadequate → D4.9

- Contractor integration issue is cultural → D7.6

D2.4 Shift Handover Inadequate

Ongoing risks, active work orders, or equipment status were not adequately transferred from one shift to the next.

Choose if: Critical information was lost during shift handover and this contributed to the incident.
→ Typical: A night shift on an oil/gas site handing over without informing the day crew about a line isolation they started but could not complete, an ongoing rock movement observation in a mine not being reported to the next shift, temporarily bypassed equipment at a power plant not being entered on the handover form, an unsafe ground condition in an unfinished excavation area on a construction site being passed over without verbal mention.

Not this if:

- No shift handover procedure exists → D9.1

- Handover information was transferred but misunderstood → D2.7

- Absence of supervision is degrading handover quality → D1.2

D2.5 Communication Not Carried Out

The message, instruction, or notification never reached its recipient.

Choose if: A planned or required communication action was never performed.
→ Typical: The required advance notification for a hazardous material delivery on an oil/gas site not being made, a team briefing before tool-box work in a mine being skipped, relevant operators not being notified of planned maintenance at a power plant, site safety rules never being explained to a newly arriving worker on a construction site.

Not this if:

- Information was transmitted but misunderstood → D2.7

- Communication channel design is the issue → D2.1 or D2.2

- The procedure does not define this communication → D9.1

D2.6 Incorrect or Incomplete Information

Outdated procedures, inaccurate equipment status information, or incorrect technical data distorted decision-making.

Choose if: The information itself was incorrect, incomplete, or out of date.
→ Typical: A valve shown as closed on a P&ID on an oil site being open in the field, unsafe equipment being approved based on the safety certification on an old equipment list in a mine, a revised circuit diagram at a power plant pointing to an incorrect intervention point, an old ground survey report on a construction site not reflecting current ground conditions.

Not this if:

- Information was correct but not communicated → D2.5

- Technical documentation not updated through change management → D2.8 and D4.3

- Information was misunderstood → D2.7

D2.7 Information Misunderstood

Information was transmitted; however, due to a language barrier, technical level mismatch, or ambiguity, it was not correctly interpreted by the recipient.

Choose if: The message was transmitted but was not correctly interpreted by the recipient.
→ Typical: A foreign-national contractor worker on an oil/gas site misunderstanding a procedure because technical instructions were communicated only in English, a safety instruction full of jargon in a mine being misinterpreted by a worker with a different education level, valve position information communicated verbally at a power plant being understood as "closed" instead of "open," an international engineering team on a construction site incorrectly converting dimensional information due to using a different measurement system.

Not this if:

- Information was never communicated → D2.5

- The information itself was incorrect → D2.6

- Language barrier is a procedural issue → A1.4

D2.8 Technical Documentation Currency

Drawings, P&IDs, or technical documents do not reflect modifications made in the field.

Choose if: Technical documents do not reflect physical changes made in the field and this contributed to the incident.
→ Typical: A modified pipeline route on an oil/gas site not being reflected in the P&ID, an electrical panel modification in a mine not being updated in the circuit diagram, a newly added bypass valve at a power plant not appearing in the system schematic, a changed foundation support location on a construction site not being updated in the structural drawings.

Not this if:

- Change management process has failed → D4.3 (MoC ineffective) — this is most often the true root cause

- Maintenance records are not current → D6.4

- Information is correct but was not communicated → D2.5

D3. TRAINING, COMPETENCE & WORKFORCE MANAGEMENT

D3.1 Training Not Provided

No training was delivered for a specific job risk or task.

Choose if: No training was genuinely provided for the relevant hazard or task.
→ Typical: Operations beginning on newly commissioned equipment on an oil/gas site without operational training being conducted, a new chemical substance being introduced in a mine without chemical hazard training being provided, relevant operators not being briefed before a critical process change at a power plant, new contractor workers starting on a construction site without site safety training being delivered.

Not this if:

- Training was provided but was ineffective → D3.2 (Training design ineffective)

- Training was provided but transfer failed → D3.3

- The task changed but training was not updated → C3.7

D3.2 Training Design Ineffective

Training was delivered; however, the content, method, or scope does not match the actual job risks and task requirements.

Choose if: Training took place but its design does not address actual task risks or requirements.
→ Typical: Generic safety training on an oil/gas site that is not task-specific failing to address specific process hazards, classroom-only training in a mine that does not reflect the dynamic conditions encountered in the field, an emergency simulation at a power plant built on outdated scenarios that do not include the current equipment configuration, contractor safety training on a construction site that does not address project site-specific risks.

Not this if:

- Training was never provided → D3.1

- Training content is sound but the instructor is ineffective → D3.3

- Knowledge deteriorated over time → D3.4

D3.3 Training Knowledge Transfer Inadequate

Training content and scope are correctly designed; however, the instructor's pedagogical approach failed to produce adequate learning in participants.

Choose if: Training content was correctly designed but the instructor's delivery method, participant engagement, or teaching skill was inadequate.
→ Typical: A highly technical expert on an oil/gas site being skilled at conveying theoretical knowledge but unable to explain it in a practical context, training conducted in a mine through a monotonous, one-way presentation that produces no lasting learning, a critical procedure at a power plant being read aloud in the classroom and never discussed, safety training on a construction site being considered complete once signatures are collected.

Not this if:

- Training design is fundamentally wrong → D3.2

- Training was never provided → D3.1

- Language barrier or personal comprehension difficulty → D2.7 or C2.3

D3.4 Knowledge Forgotten

Adequate refresher training has not been conducted for infrequently performed tasks or emergency procedures.

Choose if: The person received training and was competent at the time; however, knowledge has significantly diminished through prolonged non-use and no refresher training was provided.
→ Typical: An emergency shutdown procedure on an oil/gas site for which no drill has been conducted for years being incorrectly applied during a real emergency, a rarely performed rescue operation procedure in a mine having degraded due to lack of practice, certified training received five years earlier at a power plant not having been refreshed, a rarely used specialized equipment procedure on a construction site having faded from memory over the years.

Not this if:

- Training was never provided → D3.1

- Competency matrix does not track these skills → D3.5

- Individual skill degradation is involved → C3.4

D3.5 Critical Competency Gaps

Workforce planning has failed to maintain the competencies required for critical safety roles.

Choose if: It can be clearly established that personnel with the competency required for a safety-critical role are not available within the organization.
→ Typical: Critical equipment periodic inspections on an oil/gas site not being conducted due to the absence of an authorized pressure vessel inspection engineer, the number of qualified blasting personnel in a mine not being sufficient to meet the production program, critical protection relay coordination tests at a power plant being conducted without a specialist, complex structural work on a construction site continuing without an adequate safety engineering complement.

Not this if:

- Individual competency assessment failed → C3.1

- Training was not provided but personnel are available → D3.1

- Contractor competency issue is involved → D7.2

D4. RISK, CHANGE & WORK CONTROL SYSTEMS (CoW)

D4.1 Risk Assessment Inadequate

Hazards were not identified or risk was incorrectly calculated; a critical scenario failed to pass through analytical processes such as JSA/HAZOP/LOPA.

Choose if: It can be clearly established that hazards were not identified or that risk was systematically misjudged.
→ Typical: A JSA on an oil/gas site failing to identify a critical fire ignition source, a rock mechanics risk in a mine being assessed with an incorrect probability and resulting in inadequate controls, an operator-automated system interaction risk not being addressed at all in a power plant commissioning JSA, the settlement risk to adjacent structures not being included in the assessment for excavation works on a construction site.

Not this if:

- Risk was identified but controls were not applied → D4.2 (Risk controls not applied)

- HAZOP/technical risk analysis was never conducted → D5.7

- Risk was not updated due to changed conditions → D4.3 (MoC ineffective)

D4.2 Risk Controls Not Applied

Control measures identified in the JSA or risk assessment were not translated into field practice.

Choose if: Controls were correctly identified at the planning stage but were not implemented in field application.
→ Typical: A detector installation for gas detection identified in a JSA on an oil site not being set up in the field, a support system defined in the work plan in a mine not being installed before work commenced, a synchronization check included in the commissioning plan at a power plant being skipped in practice, protective barriers specified on the work permit form on a construction site not being installed before work began.

Not this if:

- Risk was never identified → D4.1

- Failure to apply controls is a permit system failure → D4.4

- It stems from absence of monitoring mechanism → D9.5

D4.3 Management of Change (MoC) Ineffective

Changes to equipment, processes, organization, or personnel were implemented without updating the risk analysis.

Choose if: A physical, procedural, or organizational change was made and the MoC process was not applied for that change.
→ Typical: A pipeline route change on an oil/gas site being implemented without updating the P&ID or renewing the risk assessment, new drilling equipment in a mine being commissioned without going through the MoC process, a control system software update at a power plant being applied without a risk assessment, a design change on a construction site being put into practice without being communicated to the field team.

Not this if:

- No change occurred and the risk assessment was inadequate from the outset → D4.1

- Technical documentation is not current → D2.8

- Competency was not updated following a task change → C3.7

D4.4 Permit to Work (PTW) System Failure

Required work permits were not obtained, or the permits that were obtained were not effective in practice.

Choose if: A required permit was not obtained, or the permit system was inadequate to provide control.
→ Typical: Welding being carried out without a hot work permit on an oil/gas site, entering a tunnel in a mine without obtaining a confined space entry permit, intervening on an electrical panel at a power plant without a high-voltage permit, excavation on a construction site beginning without obtaining a line break permit.

Not this if:

- A permit was obtained but LOTO was skipped → D4.5 (Energy isolation/LOTO ineffective)

- Procedure violation is an individual decision → A1.1 or A1.3

- Permit system design is inadequate → D9.2

D4.5 Energy Isolation (LOTO) Ineffective

The LOTO procedure was applied incompletely or inadequately; energy isolation was not verified.

Choose if: A LOTO procedure was applied but was incomplete or inadequate, and this resulted in exposure to an energized system.
→ Typical: Hydraulic energy sources not being isolated after electrical lockout on an oil/gas site, a lock being applied in a mine but energy isolation not being verified, only one of multiple energy sources being isolated at a power plant, a single-source lockout procedure on a construction site not covering a multi-source energy system.

Not this if:

- LOTO was entirely skipped → D9.5 (Monitoring/audit inadequate)

- The work permit does not require LOTO → D4.4

- No LOTO procedure exists at all → D9.1

D4.6 Temporary Controls Became Permanent

Bypass or control measures that were initiated as temporary have been adopted as long-term solutions without the MoC process being applied.

Choose if: A temporary measure has been treated as permanent for an extended period and this was accepted without going through the MoC process.
→ Typical: A "temporary" pressure bypass line on an oil/gas site operating unchanged for months, a temporary ventilation arrangement set up during a breakdown in a mine continuing as if it were a permanent solution, a temporary cable installation at a power plant being used for years as a permanent line, a temporary foundation support system on a construction site not being removed before the structure is completed.

Not this if:

- MoC process identified this change → D4.3

- Deferred maintenance made the temporary solution necessary → D6.6

- Temporary solution becoming permanent was a management decision → D1.9

D4.7 Scope of Work Changed

Work exceeded the defined scope and no new risk assessment or permit was obtained for this expansion.

Choose if: Work went beyond the boundaries of the original permit or risk assessment.
→ Typical: A permit obtained for a valve inspection on an oil/gas site evolving into a pipeline modification in the field, a permit obtained for limited excavation in a mine exceeding scope and extending into a different area, a control system maintenance task at a power plant transforming into a software modification during execution, a routine inspection visit on a construction site turning into an on-site intervention.

Not this if:

- Scope change was authorized and approved → D4.3 (MoC)

- An individual rule violation is involved → A1.1

- Permit system cannot perform scope control → D4.4

D4.8 Work Area Left Unsafe

After work was completed, barriers, guards, or isolations were not reinstated.

Choose if: Required safety measures were not restored to their original state after work was completed.
→ Typical: A protective cover not being reinstated after maintenance is completed on an oil/gas site, warning signs not being removed after a repair in a mine, an interlock being left in the disabled state after testing at a power plant, an excavation area on a construction site being left without barriers after work concluded.

Not this if:

- Procedure compliance is an individual violation → A1.1 or A3.6

- Absence of supervision prevents detection → D1.2

- MoC does not define reinstatement requirements → D4.3

D4.9 Simultaneous Operations (SIMOPS) Inadequate

Planning and coordination gaps existed between multiple operations being conducted at the same time.

Choose if: The interaction between simultaneously conducted operations was not adequately planned.
→ Typical: Hot work and a hydrocarbon release-risk operation continuing simultaneously in the same area on an oil/gas site, blasting and personnel transport operations being run on an overlapping schedule in a mine, online and offline equipment maintenance being planned simultaneously at a power plant, multiple disciplines performing simultaneous high-risk work on the same floor of a construction site.

Not this if:

- Coordination is at individual team level → A4.9

- Inter-organizational communication issue is primary → D2.3

- Permit system cannot control simultaneity → D4.4

D4.10 Barrier Management Inadequate

The status of critical protection layers is not being monitored, bypassed barriers are not being detected, and non-functional barriers are going unidentified.

Choose if: The status of critical safety barriers is not being systematically monitored, or bypasses are continuing undetected.
→ Typical: The bypass status of a safety instrumented system (SIS) on an oil/gas site going undetected for months, detection of critical protection relays being out of service in a mine being delayed, multiple independent protection layers simultaneously becoming non-functional at a power plant without being captured by the monitoring system, a fire detection system on a construction site being left in out-of-test status without being recorded.

Not this if:

- A single device failure is involved → B1.x

- MoC authorized the bypass → D4.3

- Inspection program has failed → D6.3

D5. ENGINEERING / DESIGN & TECHNICAL SYSTEMS

D5.1 Technical Design Errors

Metallurgy, structural load capacity, material selection, or process design was inappropriate for the task or environment.

Choose if: The design itself was technically incorrect and this error is among the direct causes of the incident.
→ Typical: Incorrect metallurgy selection for a pipeline in contact with corrosive fluid on an oil/gas site, a structural support system design in a mine that does not meet actual load conditions, a pipe connection design at a power plant based on an incorrect thermal expansion coefficient calculation, a foundation design on a construction site that incorrectly models ground conditions.

Not this if:

- Design was correct but installation was wrong → D5.4 (Installation error)

- Design inputs were incorrect → D5.2

- Ergonomic design is the issue → D5.3

D5.2 Design Inputs Incorrect

The design was based on outdated standards, incomplete data sets, or incorrect operational parameters.

Choose if: The inputs underpinning the design (data, standards, parameters) were incorrect.
→ Typical: Equipment on an oil/gas site selected based on a design pressure that incorrectly reflects the actual process pressure profile, a structural calculation in a mine performed using outdated ground survey data, a heat exchanger at a power plant designed to an outdated standard that excludes the operational temperature range, a structural calculation on a construction site conducted to an outdated seismic code.

Not this if:

- Design inputs were correct but the design was flawed → D5.1

- HAZOP/LOPA failed to identify this input error → D5.7

- Technical documentation is not current → D2.8

D5.3 Ergonomic Design Errors

The human-machine interface, control layout, or workspace design facilitates human error or makes safe working difficult.

Choose if: The design systematically makes safe use or correct operation difficult.
→ Typical: An emergency shutdown button being placed in an invisible corner on an oil/gas site, controls with different functions being designed in the same color and shape in a mine control room, a critical valve at a power plant being positioned in a confined space with no maintenance access, hazardous material storage containers of similar appearance on a construction site being placed side by side without distinguishing markings.

Not this if:

- Individual error is unrelated to design → A4.5 or A4.6

- Workspace conditions are a maintenance issue → B4.x

- Labelling or marking is inadequate → B4.6

D5.4 Installation Error

Design requirements were not followed during the construction or installation phase.

Choose if: The design was correct but technical requirements were not met during the implementation phase.
→ Typical: The specified torque value not being applied on an oil/gas site resulting in a flange leak, a support system in a mine being installed at different spacings from the design, cable insulation requirements not being met at a power plant, the concrete mix ratio on a construction site deviating from the design specification.

Not this if:

- The design itself is flawed → D5.1

- Installation oversight failed → D5.5 or D6.3

- Contractor quality control is inadequate → D7.5

D5.5 Operational Readiness (PSSR) Gaps

Pre-startup or pre-commissioning verification processes were skipped or were inadequate.

Choose if: Whether a system or piece of equipment was ready for real operations was not systematically verified before commissioning.
→ Typical: A new pipeline on an oil/gas site being commissioned without a pressure test, a newly installed ventilation system in a mine being put into operation without its effectiveness being measured, a newly commissioned system at a power plant being started without functional testing of its safety instrumentation, a load-carrying system on a construction site being opened for use without a load test after assembly.

Not this if:

- Installation is incorrect → D5.4

- Inspection program is functioning but missed the test → D6.3

- HAZOP did not identify commissioning risks → D5.7

D5.6 Early-Stage Monitoring Inadequate

The initial operating performance consistency of newly commissioned equipment or systems was not verified.

Choose if: The performance of new equipment or a system was not systematically monitored during the initial operating period, and this concealed a hidden problem.
→ Typical: Vibration trends of a new pump installation on an oil/gas site not being measured during the first month, deformation monitoring not being conducted after new support system installation in a mine, thermal performance of a new heat exchanger at a power plant not being tracked after commissioning, deflection measurements of newly assembled steel structures on a construction site not being taken under first load.

Not this if:

- Long-term inspection program has failed → D6.3

- PSSR testing was skipped → D5.5

- Design does not define monitoring requirements → D5.1

D5.7 Technical Risk Analyses (HAZOP/LOPA) Missing

Critical hazard scenarios were not subjected to systematic technical analysis.

Choose if: Required HAZOP, LOPA, FTA, or equivalent technical risk analysis was either never conducted or missed a critical scenario.
→ Typical: A new process unit on an oil/gas site being commissioned without a HAZOP being conducted, a new explosive use scenario in a mine not being subjected to risk analysis, safety function requirements at a power plant not being verified through LOPA, a complex lifting operation on a construction site proceeding without a technical risk analysis.

Not this if:

- Analysis was conducted but findings were not applied → D4.2

- Analysis was conducted but did not detect design errors → D5.1

- A change was made without MoC → D4.3

D5.8 Inherently Safer Design (ISD) Not Considered

ISD principles such as minimization, substitution, or limitation were not taken into account during the evaluation phase.

Choose if: ISD options such as reducing hazardous inventory, substituting a hazardous substance, or limiting a hazardous process were not evaluated at the design stage.
→ Typical: Large storage capacity being designed on an oil/gas site without examining the option to reduce hazardous chemical inventory, a safer alternative explosive type in a mine not being considered, a high-pressure system alternative at a power plant not being evaluated before continuing with a standard high-pressure solution, a hazardous tall-lift operation on a construction site being approved without evaluating modular ground-level assembly alternatives.

Not this if:

- Risk analysis was conducted but a non-ISD control was selected → evaluate together with D4.1

- Design inputs are incorrect → D5.2

- Technical infeasibility exists but is not documented → design documentation issue

D6. MAINTENANCE, INSPECTION & ASSET INTEGRITY

D6.1 Maintenance Strategy Inadequate

The maintenance approach remains reactive; preventive, predictive, or risk-based strategies are not being applied.

Choose if: The maintenance strategy relies only on post-failure intervention rather than proactively preventing failures.
→ Typical: Critical pumps on an oil/gas site being replaced only when they break down and operated without a predictive monitoring program, conveyor systems in a mine being taken in for maintenance only when complaints arise rather than based on failure history, no vibration monitoring program existing for critical rotating equipment at a power plant, only a reactive maintenance policy being applied to lifting equipment on a construction site.

Not this if:

- A strategy exists but execution fails → D6.2 (Maintenance execution inadequate)

- Maintenance deferral has been normalized → D6.6

- Inspection program is inadequate → D6.3

- No asset integrity management system exists → D6.7

D6.2 Maintenance Execution Inadequate

The maintenance strategy is correctly defined; however, the quality of field-level execution is inadequate. Procedures were not followed, steps were skipped, or standards were not met.

Choose if: A maintenance plan existed and was applied on time; however, the technical execution quality was inadequate.
→ Typical: A flange make-up procedure on an oil/gas site being completed without applying the correct torque values, conveyor chain maintenance in a mine having lubrication steps skipped, a heat exchanger maintenance at a power plant where the tube cleaning procedure was only partially carried out, hydraulic fluid level checks being skipped during crane periodic maintenance on a construction site.

Not this if:

- Maintenance strategy is fundamentally wrong → D6.1

- Maintenance was not performed on time → D6.6 (Deferred maintenance normalization)

- Technical personnel competency is inadequate → C3.1 or D3.5

- Inspection failure is a separate issue → D6.3

D6.3 Inspection / Calibration Inadequate

The inspection or calibration program failed to detect equipment deterioration or measurement deviation in a timely manner.

Choose if: Failure of the inspection or calibration program resulted in a hidden deterioration or deviation going undetected.
→ Typical: Pipe wall thickness measurements on an oil/gas site not being taken at sufficient frequency, causing corrosion progression to be missed, fatigue cracks in load-bearing connectors in a mine not being detected by the NDT program, a protection relay test program at a power plant being continued despite having expired, crane lifting capacity verification on a construction site being excluded from the annual program.

Not this if:

- Inspection program exists but strategy is wrong → D6.1

- Action was not taken based on inspection findings → D1.3 or D6.6

- No calibration procedure exists → D9.1

- Early-stage monitoring is the issue → D5.6

D6.4 Maintenance Records Inadequate

Maintenance history, equipment status, or records of previous findings are incomplete, inaccessible, or unreliable.

Choose if: Incomplete or unreliable maintenance records prevented a correct decision from being made or masked a hidden problem.
→ Typical: A temporary repair made during a previous maintenance intervention on an oil/gas site not being recorded, leaving the next crew without this information, irregular service history records in a mine preventing a recurring failure pattern from being identified, the last inspection date of critical equipment at a power plant not having been entered into the system, scaffold modifications on a construction site not being recorded, making safe load capacity uncertain.

Not this if:

- Maintenance execution is inadequate → D6.2

- Technical documentation issue is broader → D2.8

- MoC is not recording changes → D4.3

D6.5 Inappropriate Maintenance Type Assigned (Wrong Maintenance Strategy)

The wrong maintenance approach has been selected for a piece of equipment without taking its criticality, failure modes, or operating conditions into account — for example, applying only a reactive or time-based preventive approach when condition-based or predictive maintenance was required.

Choose if: The maintenance strategy assigned to the equipment is fundamentally mismatched with its failure characteristics, criticality level, or operating profile, and this mismatch — rather than a failure of execution — is the primary factor that allowed deterioration to go undetected.
→ Typical: On an oil/gas site, a run-to-failure approach being applied to a high-criticality safety valve when condition monitoring was required, resulting in a hidden failure mode going undetected until the device was demanded; on a mine site, time-based preventive maintenance intervals being set far too infrequently for heavily utilized conveyor drive components despite available failure history data; at a power plant, a vibration-based predictive maintenance program never being applied to critical rotating machinery despite recurring bearing failure patterns; on a construction site, lifting equipment being maintained only after reported breakdown with no periodic condition assessment, allowing structural fatigue to accumulate undetected.

Not this if:

- A maintenance strategy is correctly defined but field execution quality is poor → D6.2 (Maintenance execution inadequate)

- The strategy is appropriate but maintenance is systematically deferred → D6.6 (Deferred maintenance normalization)

- No maintenance strategy exists at all → D6.1 (Maintenance strategy inadequate)

- Inspection or proof testing is absent or inadequate → D6.3 (Inspection/calibration inadequate)

- Safety critical equipment has not been identified as a distinct category → D6.8 (SCE not identified or prioritized)

- Deterioration was detected by inspection but no action was taken → D1.3 (Accountability deficit) or D6.6

- Equipment failure resulted from long-term material degradation without strategy being the cause → B2.7 (Equipment integrity loss — corrosion/erosion/fatigue)

D6.6 Deferred Maintenance Normalization

Known maintenance needs or identified defects are being systematically deferred under production or cost pressure, and this deferral has become accepted as normal.

Choose if: Deferral has become the norm in maintenance decision-making and known deterioration has been allowed to continue without intervention.
→ Typical: A pump with identified increasing vibration on an oil/gas site being kept running "until the next planned shutdown," with this decision being repeated multiple times, a conveyor wear finding in a mine being deferred week after week, an identified corrosion finding at a power plant resulting in pipeline repair being postponed for months, a scaffold component with a detected crack on a construction site continuing to be loaded without replacement.

Not this if:

- Deferral is a one-time event with management decision → acceptable risk management

- Maintenance strategy is reactive → D6.1

- Normalization of deviance is a broader cultural issue → evaluate together with D1.5

D6.7 Asset Integrity Management System Inadequate

No integrated system exists — or the existing system is ineffective — for monitoring, prioritizing, and managing the long-term integrity of critical equipment.

Choose if: The absence or inadequacy of an asset integrity management system resulted in deterioration in critical equipment going undetected for an extended period.
→ Typical: A pipeline integrity management program on an oil/gas site excluding critical corrosion points from its monitoring scope, risk-based inspection (RBI) methodology never being applied in a mine, no life cycle management plan existing for critical rotating equipment at a power plant, absence of an integrated integrity monitoring system for crane and lifting equipment on a construction site.

Not this if:

- Inspection program exists but specific tests fail → D6.3

- Maintenance strategy is inadequate → D6.1

- This requirement was never identified at the design stage → D5.1 or D5.7

D6.8 Safety Critical Equipment (SCE) Not Identified or Prioritized

Equipment that directly performs a safety function — including safety valves, SIS components, fire and gas detection systems, emergency shutdown systems, and similar — has not been explicitly identified, and no separate management, testing, and verification regime has been established for this equipment.

Choose if: Safety critical equipment has never been formally defined as a distinct category within the asset management system, and no SCE-specific performance standards, proof test program, or verification regime exists — or the existing regime is so incomplete that it fails to ensure the functional integrity of safety barriers.
→ Typical: On an oil/gas site, pressure safety valves and ESD valves never having been listed in a formal SCE register, resulting in these items receiving no higher priority than standard process equipment in the maintenance plan; on a mine site, fire and gas detection systems not being included in any proof test program and their functional integrity never being periodically verified; at a power plant, SIS trip functions being maintained under a general work order system without defined performance standards or required response times; on a construction site, emergency stop systems on cranes and lifting equipment never having been identified as safety critical and consequently receiving no documented testing or inspection regime.

Not this if:

- An SCE register exists but specific test or inspection execution fails → D6.3 (Inspection/calibration inadequate)

- SCE is identified but its maintenance strategy is inadequately designed → D6.1 (Maintenance strategy inadequate)

- SCE is identified but proof test findings are not acted upon → D6.6 (Deferred maintenance normalization)

- The safety function was never identified at the HAZOP/LOPA stage → D5.7 (HAZOP/LOPA inadequate)

- Barrier management is not linked to SCE performance → D4.9 (Barrier management inadequate)

- SCE documentation and specifications are outdated → D2.8 (Technical documentation inadequate)

- A specific safety device was bypassed as an individual decision → A3.5 (Safety device bypassed)

- A safety device was previously removed and not reinstated → A3.6 (Guards previously removed)

- A protective device was present but failed to perform → B1.2 (Defective protective device) or B1.5 (Alarm/safety device failed)

D7. CONTRACTOR AND SUPPLY CHAIN MANAGEMENT

D7.1 No Contractor Prequalification Process

No standardized prequalification process exists within the organization to evaluate a contractor firm's safety performance, management system, legal compliance, and technical competency before it is admitted to site.

Choose if: No formal prequalification system, criteria, or approval mechanism has ever been established to evaluate contractors before they begin work on site.
→ Typical: On an oil/gas site, a contractor being started directly without any HSE record, accident statistics, or training documentation being requested, simply because it is a "familiar company"; on a mine site, contractor selection being based solely on price quotation with no safety performance scrutiny whatsoever; at a power plant, a subcontractor being engaged for critical maintenance work for the first time without undergoing any prior evaluation; on a construction site, subcontractors being admitted to site without a safety management system certificate being requested.

Not this if:

- A process exists but is ineffective → D7.2 (Contractor prequalification process ineffective)

- A process exists, the contractor failed, but was admitted anyway → D7.3 (Use of unapproved contractor)

- Contractor selection criteria do not give adequate weight to safety → D7.4 (Contractor selection ineffective)

- Contractor was admitted but individual worker competencies were not verified → C3.1

- An individual contractor worker committed a rule violation → A1.1

D7.2 Contractor Prequalification Process Ineffective

A formal prequalification process exists; however, its scope, criteria, or quality of application is insufficient to surface the contractor's genuine safety weaknesses.

Choose if: A prequalification process exists procedurally, but the evaluation remains at the level of document review only, fails to measure actual HSE capacity, and this inadequacy contributed to the event.
→ Typical: On an oil/gas site, contractor evaluation consisting solely of an insurance certificate check and review of a few procedure documents, with no scrutiny of actual accident history or HSE culture; on a mine site, desk-based document review being accepted without field audit or reference checks; at a power plant, the signing of a contractor approval form being accepted as proof of competency; on a construction site, TRIR or lost-time incident statistics never being requested from the contractor.

Not this if:

- No process exists at all → D7.1 (No contractor prequalification process)

- A process exists, a weakness was known, but the contractor was admitted anyway → D7.3

- Contractor selection criteria are a design-level problem → D7.4

- Individual competency assessment is inadequate → C3.1

- Theoretical evaluation fails to capture practical deficiencies → C3.2

D7.3 Use of Unapproved Contractor

A contractor that failed prequalification, was known not to meet the required criteria, or was never evaluated at all, was deliberately onboarded through a conscious management decision.

Choose if: It can be clearly established that the contractor's non-conformance was known, yet it was admitted to site under cost, schedule, or "no other option" pressure.
→ Typical: On an oil/gas site, a contractor that does not meet prequalification criteria being admitted on the grounds of "urgent work, no other company available"; on a mine site, HSE requirements being consciously relaxed under schedule pressure and an inadequate contractor being approved; at a power plant, a planned contractor cancelling at the last minute and an emergency contractor being mobilized without any evaluation; on a construction site, a project manager knowingly approving a contractor with incomplete safety documentation in order to protect the cost target.

Not this if:

- No process exists at all → D7.1

- A process exists but is inadequate and the weakness was therefore not detected → D7.2

- The manager personally committed a violation → evaluate together with A1.3 (Manager/supervisor rule violation)

- The accountability mechanism does not challenge this decision → D1.3

- Production/schedule pressure is a systemic organizational problem → D1.4

D7.4 Contractor Selection Ineffective

During contractor selection, criteria such as price, speed, commercial terms, and relationships were prioritized; the contractor's safety performance and culture were not adequately weighted in the selection decision.

Choose if: Safety criteria are either entirely absent from the design of the contractor selection process or carry a meaninglessly low weight, and this design gap contributed to the event.
→ Typical: On an oil/gas site, safety performance being represented by only a token weighting in tender scoring, with TRIR, lost-time rates, or HSE system audit results not constituting formal selection criteria; on a mine site, the lowest-priced bid being automatically selected without any safety evaluation; at a power plant, no minimum HSE score threshold existing in the contractor selection matrix; on a construction site, a contractor with a poor previous safety record being reselected due to a price advantage.

Not this if:

- No prequalification process exists at all → D7.1

- A prequalification process exists but is ineffective → D7.2

- Selection criteria were adequate but were deliberately disregarded → D7.3

- Contractor monitoring is absent → D7.5

- Contractor culture integration is inadequate → D7.6

D7.5 Contractor Work Process Monitoring Absent or Ineffective

No systematic process is defined for monitoring and auditing contractor activities in the field; existing audits are inadequate in terms of method, scope, frequency, and feedback; and deviations are not corrected even when identified.

Choose if: No systematic field monitoring or audit process exists for contractor activities, or existing audits are inadequate in method and scope, and this situation contributed to the event.
→ Typical: On an oil/gas site, no regular site walks, job-site observations, or HSE audits being conducted on the contractor, with only the work permit form being signed; on a mine site, a contractor crew operating outside the permit system for months without detection; at a power plant, contractor PPE compliance never being observed and no enforcement or improvement process being initiated despite identified non-conformances; on a construction site, subcontractors never being subjected to periodic safety evaluation.

Not this if:

- Contractor selection is inadequate → D7.1, D7.2, or D7.4

- A general supervision problem exists (beyond contractors) → D1.2 (Inadequate supervision)

- Audits are being conducted but finding follow-up is absent → D1.3 (Accountability deficit)

- An audit procedure has never been defined → D9.1

- The monitoring mechanism was not established in the contract → contract management issue

D7.6 Weak Contractor Safety Culture Integration

The contractor firm operates according to its own rules and culture and has not fully integrated into the facility's safety standards and expectations; the contract, orientation, and on-site oversight do not adequately support this integration.

Choose if: The contractor's organizational safety values, attitudes, or behavioral norms fundamentally conflict with the facility's safety culture, and this misalignment directly contributed to the event.
→ Typical: On an oil/gas site, a contractor placing its own PPE standards ahead of facility standards and treating facility procedures as mere "suggestions"; on a mine site, contractor workers tending to bypass the main contractor's permit system as "unnecessary bureaucracy"; at a power plant, contracts being focused solely on speed and cost with HSE performance never linked to any reward or sanction mechanism; on a construction site, contractor orientation remaining at a signature-collection formality level with no genuine transfer of safety culture taking place.

Not this if:

- Integration training and orientation were never provided → D7.5 or D3.1

- Contractor performance monitoring is absent → D7.5

- A general organizational culture problem is the primary issue → D1.x

- Absence of psychological safety also affects contractor workers → D1.11

D8. PROCUREMENT, MATERIAL HANDLING, AND MATERIAL CONTROL

D8.1 Incorrect Product Ordered

The person or team creating the order inadvertently ordered the wrong product or material with the wrong technical specification.

Choose if: It can be clearly established that an incorrect product was requested during the ordering process due to an erroneous or incomplete specification communicated to the supplier, a wrong product code, a data entry error, or an unsupervised order modification.
→ Typical: On an oil/gas site, an incorrectly stated pressure class in the technical specification transmitted to the supplier resulting in an undersized valve being ordered; on a mine site, an incorrect product code on a purchase form resulting in safety equipment of a different specification being delivered; at a power plant, an engineering revision being closed without updating the purchase request form and a product being ordered to the old specification; on a construction site, the material grade being entered incorrectly (S235 instead of S355) on a hastily prepared order form.

Not this if:

- The correct product was ordered but the wrong product was delivered → D8.2 (Incorrect product received)

- The specification was deliberately changed and this constitutes a conscious violation → A1.1 or A1.3

- Design inputs were wrong from the outset → D5.2 (Erroneous design inputs)

- The procurement evaluation system failed → evaluate with the relevant D8.x category

D8.2 Incorrect Product Received

The correct product was ordered; however, the supplier sent the wrong product or substituted it; this error was not detected during the receiving and acceptance process.

Choose if: A conforming product was ordered, a different product was delivered, and this discrepancy was not detected prior to use.
→ Typical: On an oil/gas site, a high-pressure-class valve being ordered but the supplier sending a low-pressure-class valve through an unauthorized lot substitution, with the discrepancy going undetected at goods receipt; on a mine site, a protective relay set being correctly ordered but a product with different threshold values being accepted without label-to-delivery-note matching being performed; at a power plant, heat exchanger tubes arriving in a different alloy from the order specification but being placed into stores without material certificates being verified; on a construction site, S235 steel section being delivered instead of S355, the discrepancy being overlooked because only a physical count — not a documentation check — was performed.

Not this if:

- The wrong product was requested at the ordering stage → D8.1 (Incorrect product ordered)

- The correct product was delivered but used incorrectly → A2.x

- No receiving procedure exists → D9.1

- Counterfeit or substandard material is involved → D8.6

D8.3 Transportation or Shipment Inadequacy (Material Damage)

Materials were physically damaged during internal or external transportation and shipment because they were not adequately protected, packaged, or handled.

Choose if: Inadequacy in the transportation or handling process compromised the physical integrity of the material and this contributed to the event.
→ Typical: On an oil/gas site, a stack of pipeline components collapsing during transport due to inadequate palletizing, scoring flange faces and compromising sealing integrity; on a mine site, sensitive sensor equipment being shipped without vibration-protection packaging, resulting in internal electronic damage; at a power plant, chemical drums tipping over during forklift operations and causing contamination; on a construction site, steel reinforcement being dragged along the ground during handling, damaging surface integrity.

Not this if:

- Material was transported correctly but degraded in storage → D8.4 (Inadequate material storage)

- No transportation procedure exists → D9.1

- Forklift or vehicle operation was an individual violation → A2.2 or A2.1

- Packaging design is a supplier-side issue → D8.1 or a supplier contract issue

D8.4 Inadequate Material Storage

Materials were not stored in accordance with the required storage conditions and consequently degraded, lost their properties, or reached a state that creates a hazard.

Choose if: Inadequate storage conditions directly compromised the integrity or safe-use properties of the material or equipment, and this contributed to the event.
→ Typical: On an oil/gas site, elastomeric gaskets being stored in an area without humidity and temperature control, degrading and failing to achieve sealing during assembly; on a mine site, shelf load limits being exceeded, causing heavy materials on an upper shelf to tip over; at a power plant, dielectric gloves being stored alongside sharp objects and chemicals in the same storeroom, causing damage; on a construction site, expired structural adhesive remaining in use because no shelf-life control was in place.

Not this if:

- Material left the store undamaged but was damaged during transport → D8.3

- No storage procedure exists → D9.1

- Storage of incompatible chemicals in proximity was missed by risk assessment → D4.1

- PPE inspection failure is the primary issue → D6.3

D8.5 Inadequate Material Labeling and Marking

Materials were labeled incorrectly, incompletely, or illegibly, or were not labeled at all; this resulted in wrong material selection, confusion, or loss of health and safety information.

Choose if: A label deficiency, inaccuracy, or illegibility directly contributed to wrong material use or failure to communicate hazard information.
→ Typical: On an oil/gas site, missing or incorrect SDS (Safety Data Sheet) labels on chemical containers resulting in the wrong chemical being used; on a mine site, the validity of an explosive material certificate being unverifiable because no lot number or expiry date was present; at a power plant, the absence of a labeling system distinguishing quarantine, approved, and rejected material resulting in a rejected part entering service; on a construction site, fallen or erased shelf labels resulting in steel of the wrong strength grade being used in the belief that it was the correct material.

Not this if:

- The label was correct but was not read → A4.2 or A4.5

- Label verification was not performed during the receiving process → evaluate together with D8.2

- The labeling system was not updated through the MoC process → D4.3

- The maintenance program does not include label upkeep → D6.2

D8.6 Counterfeit, Suspect, or Substandard Items / Use of Unapproved Parts (CSSI)

A part or material with a fraudulent certification, non-standard manufacture, or unapproved supplier has been introduced into the system, directly creating a safety vulnerability.

Choose if: It has been confirmed, or is strongly suspected, that the equipment or material used is a product that does not meet its declared specifications, is undocumented, is counterfeit, or was sourced from an unapproved supplier.
→ Typical: On an oil/gas site, the use of a flange bearing a counterfeit API certificate leading to a pipeline leak under high pressure; on a mine site, a counterfeit safety rope manufactured from substandard material failing well below its nominal capacity under load; at a power plant, a circuit breaker bearing a counterfeit CE mark failing to meet the standard short-circuit interrupting capacity in practice; on a construction site, steel reinforcement being used whose actual strength class is significantly below the declared S355 value.

Not this if:

- The product was ordered in the correct grade but the wrong product was sent → D8.2

- Receiving verification failed and counterfeiting was not known → evaluate together with D8.5

- Supplier evaluation did not identify this risk → D8.1

- Storage conditions degraded performance → D8.4

D9. PROCEDURES, STANDARDS & COMPLIANCE SYSTEMS (SPP)

D9.1 No Procedure for Task

No written procedure, instruction, or standard exists for the task or activity being performed.

Choose if: Despite the task being defined, no procedure or work instruction required to carry it out has ever been created.
→ Typical: No written commissioning procedure existing for newly commissioned equipment on an oil/gas site, no work instruction existing for a rarely performed specialized drilling operation in a mine, no written steps existing for a critical switching operation at a power plant that requires a partly automated, partly manual combined procedure, no lifting plan or rigging procedure having been prepared for a specialized lifting operation on a construction site.

Not this if:

- Procedure exists but cannot be understood → A1.4

- Procedure exists but cannot be applied → A1.5

- Procedure quality needs improvement → D9.2

- Procedure is not current → D9.3

D9.2 SPP Development Ineffective

A procedure or standard exists; however, its content quality is not adequate for safe task execution. Steps are missing, ambiguous, or based on unrealistic assumptions.

Choose if: A procedure exists but its content cannot provide the detail or technical accuracy required for safe task execution.
→ Typical: An isolation procedure on an oil/gas site that simply says "close the valve" without specifying which valve, in what sequence, and with what verification steps, a post-blast re-entry procedure in a mine that does not clearly define the waiting time, a commissioning procedure at a power plant that says "verify the system" without specifying verification criteria, a scaffold installation instruction on a construction site that does not define connection frequency based on load capacity.

Not this if:

- No procedure exists at all → D9.1

- Procedure is inaccessible or unknown → D9.3

- Procedure is well written but cannot be applied in the field → D9.4

D9.3 SPP Communication / Access Ineffective

A procedure exists and is well written; however, workers cannot access it, are unaware of it, or cannot reach the current version.

Choose if: A procedure exists and is of quality, but workers do not know about it, cannot find it, or cannot access the current version.
→ Typical: A critical procedure on an oil/gas site existing only on a central server and being unreachable when there is no field computer access, a revised evacuation procedure in a mine being updated only digitally and not distributed to field workers, a newly published emergency shutdown procedure at a power plant not being communicated to all shift operators, a procedure booklet on a construction site prepared only in English making it inaccessible to foreign-national workers.

Not this if:

- Procedure cannot be understood → A1.4

- Procedure does not exist → D9.1

- Training does not include this procedure → D3.1

D9.4 SPP Applicability Weak

The procedure is correctly written on paper; however, it is not practically possible to apply it in the field. It is based on unrealistic assumptions in terms of time, access, equipment, or physical conditions.

Choose if: A procedure exists and is known, but it is not practically possible to apply it under field conditions.
→ Typical: An isolation valve specified in a procedure on an oil/gas site not physically existing in the field, a step estimated at 30 minutes in a mine exceeding three hours in the field and causing workers to find shortcuts, a step requiring two people in a procedure at a power plant having to be performed by a single person on a one-person shift, the minimum working distance specified in a procedure on a construction site being impossible to apply due to physical space constraints.

Not this if:

- Procedure cannot be understood → A1.4

- No procedure exists → D9.1

- Procedure is inapplicable due to design errors → D5.3

- An individual violation is involved → A1.1

D9.5 Monitoring / Audit Inadequate

The systematic observation, audit, or verification mechanism that should confirm whether procedures are actually being followed in the field is non-functional or absent.

Choose if: The monitoring or audit mechanism that should verify procedure compliance at field level does not exist or is ineffective.
→ Typical: Whether the permit to work system is being applied on an oil/gas site not being checked through periodic audits, PPE usage in a mine not being observed in the field, LOTO compliance at a power plant being accepted without a verification tour, scaffold installation standard compliance on a construction site never being audited.

Not this if:

- Audits are conducted but findings are not followed up → D1.3 (Lack of accountability)

- Leadership visibility is the issue → D1.8

- No audit procedure has ever been designed → D9.1

D9.6 Regulatory Compliance Management Inadequate

Legal and regulatory requirements are not being systematically tracked or implemented within the organization.

Choose if: Applicable legal or regulatory requirements were not tracked or systematically implemented within the organization.
→ Typical: Operations on an oil/gas site continuing with an outdated ATEX zoning document, a periodic risk assessment in a mine not being renewed beyond the regulatory deadline, a newly enacted electrical safety regulation at a power plant not being reflected in procedures, mandatory training certifications on a construction site being allowed to expire without tracking legal renewal dates.

Not this if:

- Regulation was known but deliberately violated → D1.3 or D1.9

- No tracking system exists → corporate governance system issue

- Legal change requires MoC process → D4.3

D10. EMERGENCY PREPAREDNESS & RESPONSE

D10.1 Emergency Planning Inadequate

The emergency plan does not cover the scenario that occurred, is not current, or has never been prepared.

Choose if: The existing emergency plan does not adequately cover the incident that occurred, or no plan exists at all.
→ Typical: An emergency plan prepared for a major gas leak scenario on an oil/gas site covering only a fire scenario, no rock fall emergency response plan existing in a mine, a major power outage emergency plan at a power plant not reflecting the latest technical changes, no emergency procedure having been prepared for a deep excavation collapse scenario on a construction site.

Not this if:

- Plan exists but no drills were conducted → D10.2

- Plan exists but equipment is inadequate → D10.4

- Communication system failed → D10.3

D10.2 Drills and Practice Inadequate

An emergency plan exists; however, regular drills have not been conducted and personnel have not had the opportunity to develop real response capacity.

Choose if: A plan exists but drills have not been conducted in practice and this deficiency negatively affected the real emergency response.
→ Typical: An evacuation plan on an oil/gas site remaining on paper for years without a drill being conducted, a mine rescue team obtaining certification without conducting drills close to real conditions, control room operators at a power plant having never practiced emergency scenarios in a simulation environment, the serious injury emergency response chain on a construction site having never been tested.

Not this if:

- No plan exists → D10.1

- Drills were conducted but lessons learned were not applied → D10.6

- Equipment was inadequate during a drill → D10.4

D10.3 Emergency Communication System Inadequate

The communication infrastructure required for alarm, evacuation, or response coordination failed or never existed.

Choose if: The communication system required during an emergency either did not function or had never been established.
→ Typical: Communications collapsing during a major incident on an offshore platform when all radio frequencies became congested, the underground-to-surface communication system losing connection during an emergency in a mine, the communication system between the crisis management center and field teams at a power plant failing during a major power outage, an evacuation announcement not reaching a high-noise machinery area on a construction site.

Not this if:

- System functioned but was inadequate → B1.4

- System never activated → B1.5

- Plan does not define communication protocol → D10.1

D10.4 Emergency Equipment Inadequate or Not Ready

First response equipment (fire suppression, rescue equipment, medical supplies) is insufficient, poorly maintained, or inaccessible.

Choose if: Equipment required for emergency response was missing, poorly maintained, or unusable.
→ Typical: A fire hydrant on an oil/gas site not being present in the area or producing no pressure, rescue room oxygen cylinders in a mine being found empty during an emergency, first aid kit supplies at a power plant having been depleted, fall rescue equipment on a construction site being locked in an inaccessible storage room.

Not this if:

- Equipment is available but how to use it is unknown → D3.1 or D10.2

- Equipment maintenance was neglected → D6.2

- Procurement issue prevented equipment from being acquired → D8.x

D10.5 Emergency Roles and Responsibilities Unclear

Who does what in an emergency has not been clearly defined, leading to delays or confusion in the response.

Choose if: Ambiguity in roles and responsibilities in emergency response delayed or prevented effective response.
→ Typical: The question of who manages the response remaining unanswered among the facility manager, HSE manager, and operations supervisor during a major gas leak on an oil/gas site, the ambiguity of the command chain between the site rescue team and incoming emergency services in a mine, operations and maintenance teams at a power plant conflicting with each other's interventions during a major breakdown, nobody knowing who the first aid responsible person is during a serious accident on a construction site.

Not this if:

- Plan is ambiguous → D10.1

- Drills did not provide the opportunity to surface these issues → D10.2

- Communication system is preventing coordination → D10.3

D10.6 Learning from Past Events Inadequate

Lessons drawn from previous incidents, near-misses, or drill outcomes were not fed back into the system as improvements.

Choose if: Findings obtained from past experience were not translated into systemic corrective action or procedure updates.
→ Typical: A root cause analysis of a similar past incident on an oil/gas site having been conducted but no corrective action being implemented, deficiencies identified after an annual drill in a mine remaining pending until the following year, industry safety bulletins at a power plant not being followed and lessons learned not being reflected in procedures, findings from a post-project safety review on a construction site not being carried forward to the next project.

Not this if:

- Reporting and learning culture issue is more general → D1.7

- Accountability mechanism is not following up findings → D1.3

- MoC process is not reflecting lessons learned into procedures → D4.3